NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

DRIVER AND

OUTPUT

SUBSYSTEMS

PACKAGE

(1).	Interfacing with Drivers and Buffers.

(2).	The Transistor as a Switch.

(3)	The MOSFET as a Switch.

(4).	The Relay.

(5).	Loudspeakers and AWDs (Audible Warning Devices)

(6)	Motors

Interfacing with Drivers and Buffers

This task is to give an introduction to Interfacing with Drivers and Buffers.

By the end of this task you will be able to:-

Know what Interfacing is and why it is needed,

Know that Drivers and Buffers are used for Interfacing,

Know why they are needed.

1).	Most processor subsystems are only capable of supplying a supplying a small amount of 	power to the output subsystem, e.g. a logic gate will only supply of approximately 5mA.

(a)	If the supply voltage to the logic gate is 9V, what is the maximum power that it can 		supply to the output subsystem?

.................................................................................................................................................

.................................................................................................................................................

(b)	The input of an output subsystem has a resistance of 100Ω at 9V.

What power must the processor subsystem supply to the output subsystem?

.................................................................................................................................................

.................................................................................................................................................

(c)	What is the difference in the power requirements?

.................................................................................................................................................

2).	To overcome this problem, the processor subsystem must be INTERFACED to the output 	subsystem so that the voltage, current and power requirements are matched.  A DRIVER or 	BUFFER circuit is used for interfacing.  These circuits will often be devices which amplify 	the current and power from the processor so that the output subsystem can work correctly.

(a)	Why does an output subsystem need to be interfaced to a processor subsystem?

.................................................................................................................................................

.................................................................................................................................................

(b)	What is the name of the circuits that are used for interfacing?

.................................................................................................................................................

(c)	How do they interface the processor and output subsystems together?

.................................................................................................................................................

.................................................................................................................................................

The Transistor as a Driver/Buffer Switch

This task is to give an introduction to the Transistor as a Driver/Buffer.

By the end of this task you will be able to:-

Know what a transistor is,

Know how it works as a driver/buffer switch,

1).	A transistor is a semiconductor device that is able to provide both power and current gain.

It can therefore be used as a driver/buffer.

This unit only considers the transistor as a switch.

(a)	(i)	Draw a diagram of the symbol used to represent a transistor.

(ii)	On the diagram, label the three terminals.

(iii)	On the diagram, show the path of the input and output currents.

(b)	An example of a low power transistor is the 2N3904.

Draw a diagram of the pin connections when viewed from the bottom.

(c)	Explain how a transistor works.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

(d)	What is the maximum voltage between the two input terminals of a transistor?

.................................................................................................................................................

(e)	(i)	What is the typical input current for a small transistor?

.................................................................................................................................................

(ii)	Estimate the typical input resistance for a small transistor

.................................................................................................................................................

.................................................................................................................................................

(f)	Draw a circuit diagram to show how you would connect a transistor to a logic gate so 		that the transistor controls a high power LED.

Remember to include the series resistors for both the transistor base and LED.

The input to the logic gate should be such that when a LDR is in the dark, the LED is 		switched on.

Advanced Work.

(g)	Construct your circuit and show the layout on the protoboard diagram below.

Label the diagram.

The MOSFET as a Driver/Buffer Switch

This task is to give an introduction to the MOSFET as a Driver/Buffer.

By the end of this task you will be able to:-

Know what a MOSFET is,

Know how a MOSFET works as a driver/buffer switch,

Know the advantages and disadvantages of a MOSFET as a driver/buffer switch.

1).	A MOSFET is a semiconductor device that is able to provide both power and current gain.

It can therefore be used as a driver/buffer.

This unit only considers the MOSFET as a switch.

(a)	(i)	Draw a diagram of the symbol used to represent a MOSFET.

(ii)	On the diagram, label the three terminals.

(iii)	On the diagram, show the path of the input and output currents.

(b)	An example of a medium power MOSFET is the IRF630.

Draw a diagram of the pin connections when viewed from the top.

(c)	Explain how a MOSFET works.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

(d)	What is the maximum voltage between the two input terminals of a MOSFET?

.................................................................................................................................................

(e)	(i)	What is the typical input current for a MOSFET?

.................................................................................................................................................

(ii)	Estimate the typical input resistance for a MOSFET.

.................................................................................................................................................

(iii)	Explain why a MOSFET does not need a series input resistor.

.................................................................................................................................................

.................................................................................................................................................

(f)	In the table below summarise the characteristics of transistors and MOSFETs as 			switches.  Use component catalogues, the Internet and the support booklet to 			complete the table.

|  | Transistor | MOSFET |
| --- | --- | --- |
| Cost |  |  |
| Current gain |  |  |
| Power gain |  |  |
| Input resistance |  |  |
| Output resistance |  |  |
| Reliability |  |  |
| Ease of use |  |  |

(g)	Draw a circuit diagram to show how you would connect a MOSFET to a logic gate 		so that the MOSFET controls a high power LED.

Remember to include the series resistors for the LED.

The input to the logic gate should be such that when a LDR is in the dark, the LED is 		switched on.

The Relay as a Driver/Buffer Switch

This task is to give an introduction to the Relay as a Driver/Buffer.

By the end of this task you will be able to:-

Know what a relay is,

Know how a relay works as a driver/buffer switch,

Know the advantages and disadvantages of a relay as a driver/buffer switch.

1).	A relay is an electromechanical device.

It can therefore be used as a driver/buffer.

(a)	(i)	Draw a diagram of the symbol used to represent a relay.

(ii)	On the diagram, label the input and output terminals.

(iii)	On the diagram, show the path of the input and output currents.

(b)	Draw a labelled diagram of a DPDT relay showing the main parts.

(c)	Explain how a relay works.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

(d)	What is the maximum voltage between the two input terminals of a relay?

.................................................................................................................................................

(e)	(i)	What is the typical input current for a relay?

.................................................................................................................................................

(ii)	Estimate the typical input resistance for a relay.

.................................................................................................................................................

(iii)	Explain how a relay is able to completely isolate the input and output circuits.

.................................................................................................................................................

.................................................................................................................................................

(iv)	Explain why a relay cannot be connected directly to a processor subsystem.

.................................................................................................................................................

.................................................................................................................................................

(v)	Explain what precautions must be taken when using a relay with 					semiconductors

.................................................................................................................................................

.................................................................................................................................................

(f)	Draw a circuit diagram to show how you would use a MOSFET to interface a relay 		to a logic gate so that the relay could control a mains operated electric heater.

The input to the logic gate should be such that when a thermistor is cold, the heater is 		switched on.  Remember to include the protection diode.

Loudspeakers and AWDs

This task is to give an introduction Loudspeakers and AWD as output subsystems.

By the end of this task you will be able to:-

Know what a loudspeaker and AWD is,

Know the differences between them.

1).	A loudspeaker is an electromechanical device that turns electrical energy into sound energy.

(a)	Draw a diagram of the symbol used to represent a loudspeaker.

(b)	Draw a labelled diagram of a loudspeaker showing the main parts.

(c)	Explain how a loudspeaker works.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

(d)	What is the typical resistance of a loudspeaker?

.................................................................................................................................................

(e)	For a loudspeaker to emit a tone, what other circuit is needed?

.................................................................................................................................................

.................................................................................................................................................

2).	An AWD is a generic name for a device that emits a tone when connected to a source of 	electricity.  Examples include bells and buzzers.

(a)	Name two different families of AWDs.

(b)	Draw the symbol for an AWD.

(c)	Explain the main difference between a loudspeaker and an AWD.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

(d)	What are the typical frequencies emitted by AWDs?

.................................................................................................................................................

(e)	What are the typical sound levels emitted by AWDs?

.................................................................................................................................................

.................................................................................................................................................

The Electric Motor

This task is to give an introduction to electric motors.

By the end of this task you will be able to:-

Know what an electric motor is,

Know how an electric motor works,

Know how to interface an electric motor to a control circuit.

1).	An electric motor is an electromechanical device that turns electrical energy into movement.

(a)	(i)	Draw a diagram of the symbol used to represent a motor.

(b)	Name two different types of electric motor.

.................................................................................................................................................

.................................................................................................................................................

(c)	State and explain two precautions that must be taken when using a motor with 			semiconductor drivers

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

Homework Assignment.

Complete the Online True/False “Driver and Output Subsystems Test”.  Print out and hand in the Score sheet.

Advanced work - Design Problem

An electronic circuit is needed that will turn on a fan when the temperature in a computer exceeds a safe temperature.

The circuit should operate from a 12V supply.

(a)	Draw a system diagram for a solution to the above problem and label the various 			subsystems.

(b)	Using a logic gate as the processor and a transistor as the driver, translate your 			system diagram into a circuit diagram.

(iii)	Convert your circuit diagram into a component layout on the diagram below.