NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

DIODES

AND THE

VOLTAGE

DIVIDER

PACKAGE

(1).	Diode symbols and characteristic curves.

(2)	LEDs.

(2).	Zener Diodes.

(3).	Voltage Dividers.

Diode Characteristics.

Diodes have already been used as LEDs and as protection devices in driver and buffer circuits.

The aim of this assignment is to provide an understanding of how the current through a diode varies with the voltage applied across the diode.

You will need to use the Support Booklet to complete this assignment.

There are many different types of diodes but the main ones that are required for this specification are:

Light Emitting Diodes,

Silicon signal diodes,  e.g. 1N4148

Silicon power diodes,  e.g. 1N4001 and

Zener diodes.

By the end of this task you will be able to:-

Know that there are different types of diodes,

Recognise the characteristic curves for different diodes,

(a).	(i)	Draw the circuit symbol for a LED.

(ii)	Draw the circuit symbol for a silicon diode.  (same for both signal and power).

(ii)	Draw the circuit symbol for a zener diode.

(b).	Construct the circuit below on your protoboard.

Use a digital voltmeter to measure the voltage across the diode.

Make measurements of the voltage across the diode when it is connected either way round in the circuit and record your results in the table below.  Replace the diode with a different type and record your measurements.  The smallest voltage measured should be entered in the "Forward Voltage" column in each case.

| Type of Diode | Forward Voltage (V) | Reverse Voltage (V) |
| --- | --- | --- |
| Red LED |  |  |
| Yellow LED |  |  |
| Green LED |  |  |
| Blue LED |  |  |
| White LED |  |  |
| Infra-red LED |  |  |
| Silicon signal diode |  |  |
| Silicon Power diode |  |  |
| 5.1V Zener diode |  |  |

(c)	Show your results graphically on the axes below.

Use the Support Booklet to explain and summarise your results.

Show your results graphically on the axes below.

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

.................................................................................................................................................

Light Emitting Diodes

LEDs have already been used as output devices.

The aim of this assignment is to provide an understanding of the uses of LEDs.

You will need to use the Support Booklet to complete this assignment.

By the end of this task you will be able to:-

Know of different uses for LEDs,

Know how to represent different numbers with seven segment displays,

Know the difference between common anode and cathode seven segment displays,

Calculate the series resistor for an LED.

1).	(a)	Examine and draw a diagram in the space below of a 7-segment display.

(b)	Explain what is meant by a common anode and a common cathode display.

.....................................................................................................................................

.....................................................................................................................................

(c)	Explain when you would use a common anode display and when you would use a 			common cathode display.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(d)	In the space below draw diagrams to show how the numbers from 0 to 9 can be 			produced by a seven segment display.

Calculation of the series resistor for a LED.

(2)	The maximum current for a LED can be found by looking in component catalogues and data 	sheets.

Consider a red LED with a forward voltage of 1.9V at a maximum current of 40mA.

The LED is to be operated from a 12V supply.

The procedure below enables the minimum value of the series resistor to be calculated.

(a)	What will be the voltage drop across the series resistor when there is a current of 			40mA passing through the LED?

.....................................................................................................................................

(b)	Use Ohm's law to calculate the value of this series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(c)	What is the nearest preferred value to your answer to (b) that is larger?

.....................................................................................................................................

(d)	Explain why the preferred value chosen MUST be the next largest.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(e)	Calculate the power dissipated in this series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

Example.

(3)	Consider a blue LED has a forward voltage of 4.2V at a maximum current of 30mA.

The LED is to be operated from a 9V supply.

Calculate a suitable series resistor.

(a)	What will be the voltage drop across the series resistor when there is a current of 			30mA passing through the LED?

.....................................................................................................................................

(b)	What current passes through the resistor?

.....................................................................................................................................

(c)	Use Ohm's law to calculate the value of this series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(d)	What is the nearest preferred value to your answer to (c) that is larger?

.....................................................................................................................................

(e)	Calculate the power dissipated in this series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “LED Series Resistor”.

Complete the assignment and then print out and hand in your Score sheet.

Simple Regulated Power Supplies using Zener Diodes

The aim of this assignment is to provide an understanding of the uses of Zener diodes.

You will need to use the Support Booklet to complete this assignment.

By the end of this task you will be able to:-

Recognise the characteristic curve for a Zener diode,

Know how a Zener diode can be used to give a constant voltage source,

Calculate the series resistor and power rating for a Zener diode.

As a result of a Zener diode having a well defined reverse breakdown voltage, they can be used to make simple but stable regulated power supplies.  A regulated power supply is one where the output voltage does not change as the input voltage or output current changes.

1).	(a)	Construct the circuit below on your protoboard.

Use a digital meter to measure the voltage across and the current through the zener diode.

(b)	Complete the table below by connecting different resistors across the zener diode.

| Resistor across Zener diode () | Voltage across Zener diode (V) | Current through Zener diode (mA) |
| --- | --- | --- |
| Open circuit |  |  |
| 10,000 |  |  |
| 4700 |  |  |
| 1000 |  |  |
| 470 |  |  |
| 100 |  |  |

(c)	Explain your results.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(d)	Explain how the Zener diode is able to keep the voltage across it stable.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(e)	Sketch on the axes below the characteristic curve for the zener diode that you have 		used in your circuit.

Label the important sections.

Calculation of the series resistor for a Zener Diode.

2)	Zener diodes are rated by their breakdown voltage and the maximum power that they can 	dissipate.  This information can be found in component catalogues and in manufacturers data 	sheets.  The Zener diodes that you have been using are rated at 5.1V and 400mW power 	dissipation.

A 5.1V stabilised power supply is needed from a car battery which gives approximately 	12V.

The procedure below enables the minimum value of the series resistor to be calculated.

(a)	With a maximum power dissipation of 0.4W (400mW), what is the maximum 			current that can pass through the zener diode?

.....................................................................................................................................

.....................................................................................................................................

(b)	What will be the voltage across the series resistor?

.....................................................................................................................................

(c)	The current passing through the Zener diode must also pass through this resistor.  			Use Ohm's law to calculate the value of the series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(d)	To ensure that the zener does not dissipate too much power the next LARGEST 			preferred value should be used.  What is this value?

.....................................................................................................................................

(e)	Calculate a suitable power rating for this resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(f)	What is the maximum current that the circuit can supply and still keep a stabilised 		voltage of 5.1V?

.....................................................................................................................................

.....................................................................................................................................

(g)	What current would pass through a short circuit connected across the zener diode?

.....................................................................................................................................

(h)	What power would be dissipated in the series resistor in this case?

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

3).	Sometimes the maximum current from the supply is specified and you will then have to 	calculate both a suitable resistor value and power rating for the zener diode.

A stabilised power supply is needed from a car battery which gives approximately 	12V.  The stabilised supply must be able to supply 100mA at 5.1V.

The procedure below enables the resistor and zener values to be calculated.

(a)	A zener diode needs a small current to always flow through it to keep a constant 			voltage across it.  For a small zener, this will typically be 10mA.

What will be the maximum current that will need to be able to pass through the zener 		diode?  (This will be when there is no load connected.).

.....................................................................................................................................

.....................................................................................................................................

(b)	What will be the voltage across the series resistor?

.....................................................................................................................................

(c)	The current passing through the Zener diode must also pass through this resistor.			Use Ohm's law to calculate the value of the series resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(d)	To ensure that the maximum stated current can be supplied, the next SMALLEST 			preferred value should be used.  What is this value?

NOTE:-  THIS IS DIFFERENT TO THE LAST EXAMPLE.

.....................................................................................................................................

(e)	Calculate a suitable power rating for this resistor.

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

(f)	Calculate a suitable power rating for the zener diode.

Remember to choose the next largest value to ensure that it does not dissipate too 			much power.

.....................................................................................................................................

.....................................................................................................................................

(g)	What current would pass through a short circuit connected across the zener diode?

.....................................................................................................................................

(h)	What power would be dissipated in the series resistor in this case?

.....................................................................................................................................

.....................................................................................................................................

.....................................................................................................................................

Design Problems.

(4)	A MP3 player requires an operating voltage of 2.5 to 3V at a maximum current of 200mA.

An adaptor is required to operate the MP3 player in a car from the car electricity supply.

The maximum voltage of the car supply is 14.2V.

(a)	Draw a circuit diagram to show how a 2.7V zener diode and a resistor can be used to 		form such an adaptor.

(b)	If the minimum zener diode current is 20mA, calculate a suitable power rating for the 		zener diode.

.....................................................................................................................................

.....................................................................................................................................

(c)	Calculate the maximum voltage across the resistor in series with the zener diode.

.....................................................................................................................................

(d)	Calculate a suitable value for this resistor and state the preferred value that you 			would use.

.....................................................................................................................................

.....................................................................................................................................

(e)	Calculate a suitable power rating for this resistor.

.....................................................................................................................................

.....................................................................................................................................

(f)	Calculate the short circuit current of this adaptor.

.....................................................................................................................................

(g)	Calculate a suitable power rating for the zener series resistor if it is to be able to 			withstand long periods of being short circuited.

.....................................................................................................................................

.....................................................................................................................................

(5)	A portable Global Positioning System (GPS) requires an operating voltage of 9V at a 	maximum current of 180mA.

An adaptor is required to operate the GPS in a car from the car electricity supply.

The maximum voltage of the car supply is 14.2V.

(a)	Draw a circuit diagram to show how a 9.1V zener diode and a resistor can be used to 		form such an adaptor.

(b)	If the minimum zener diode current is 10mA, calculate a suitable power rating for the 		zener diode.

.....................................................................................................................................

.....................................................................................................................................

(c)	Calculate the maximum voltage across the resistor in series with the zener diode.

.....................................................................................................................................

(d)	Calculate a suitable value for this resistor and state the preferred value that you 			would use.

.....................................................................................................................................

.....................................................................................................................................

(e)	Calculate a suitable power rating for this resistor.

.....................................................................................................................................

.....................................................................................................................................

(f)	Calculate the short circuit current of this adaptor.

.....................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Zener Series Resistor”.

Complete the assignment and then print out and hand in your Score sheet.

Voltage Dividers

The aim of this assignment is to provide an understanding of the Voltage Divider.

You will need to use the Support Booklet to complete this assignment.

By the end of this task you will be able to:-

Recognise the voltage divider circuit,

Know how a voltage divider can be used with resistive sensors,

Carry out calculations on voltage divider circuits.

1).	A voltage divider circuit is required for most applications using a resistive input transducer 	and also when a different voltage is to be produced from a fixed power supply.

The general circuit is shown below.

(a)	Set up a voltage divider circuit on your protoboard with both resistors equal to 10k 		and Vin connected to a 9V battery.

Complete the table below, but leave the Calculation column

| R1 | R2 | Vin | Vout | V ratio | R ratio | Calculation |
| --- | --- | --- | --- | --- | --- | --- |
| 10kΩ | 10kΩ |  |  |  |  |  |
| 4.7kΩ | 10kΩ |  |  |  |  |  |
| 10kΩ | 4.7kΩ |  |  |  |  |  |
| 1kΩ | 10kΩ |  |  |  |  |  |
| 10kΩ | 1kΩ |  |  |  |  |  |

(b)	What is the relationship between the input/output voltages and the two resistors?

.................................................................................................................................................

.................................................................................................................................................

(c)	Suggest a "ratio" way to predict the output voltage from a voltage divider.

.................................................................................................................................................

.................................................................................................................................................

(d)	The formula for a voltage divider is

NOTE:-  This formula is NOT on the Data Sheet

Use this formula to complete the Calculation column in the table in section (a).

Advanced Work

1).	(a)	On your circuit board, replace R1 with an LDR and R2 with a 10kΩ resistor.

Describe what will happen to Vout when the LDR is taken from the light into dark.

.........................................................................................................................................

.........................................................................................................................................

(b)	If the position of the resistor and the LDR were swapped (LDR now at the bottom), 		describe now what would happen to the voltage going to the processor when the 			LDR is taken from the light into the dark.

.........................................................................................................................................

.........................................................................................................................................

Homework Assignments.

Complete the Homework Exercise “Voltage Divider”.

After entering your name, choose from either the Level 2 (GCSE) or Level 3 (AS) tests.

Complete the assignment and then print out and hand in your Score sheet.