NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

# ELECTRONICS

INDUCTION

PACKAGE

(1)	Electronic systems.

(2).	Using a Protoboard.

(3).	Basic safety.

(4)	LEDs and Resistors

(5)	ICs - A Basic Water Alarm.

(6).	How to Solder including - The Soldering Test.

Advanced task.

(7).	A Logic Probe.

Electronic Systems

This task is to give an introduction to Electronic Systems.

Use the Support Booklet to carry out the following tasks.

By the end of this task you will be able to:-

Know what an Electronic system is,

Be able to identify Electronic systems,

Know that an Electronic System consists of    Inputs, Processors Outputs and Feedback.

1).	All electronic systems consist of four essential sections or subsystems:-

Input:- ......................................................................................................................................

Processor:- ...............................................................................................................................

Output:- ....................................................................................................................................

Feedback:- ................................................................................................................................

2).	Complete the table below and suggest three additional systems.

| System | Input | Processor | Output | Feedback |
| --- | --- | --- | --- | --- |
| Security lamp |  |  | Lamp |  |
| P.A. system | Microphone |  |  |  |
| CD player |  |  |  |  |
| Electric heater |  |  |  |  |
| Radio |  |  | Loudspeaker |  |
| Smoke alarm |  |  |  |  |
| Burglar alarm |  |  |  |  |

Summary

All electronic systems can be broken down into ................................, consisting of .......................... sensors, processors and output ..............................  Many also use feedback so that the output can control the ..........................................

Using Protoboard.

This task is to give an introduction to Protoboards.

By the end of this task you will be able to:-

Know what a Protoboard is,

Know how the ‘holes’ on a protoboard are connected,

Know how to wire a circuit on a protoboard.

The best way to learn electronics is to build circuits and investigate how they operate.

To enable circuits to be built quickly, Push-In or Solderless Breadboards have been developed.

These often have many different commercial names and shapes but they all work on the same principle, that of the wires of the components being pushed through holes in a plastic case and held tightly by a metal spring.

The general name used for all of these boards is PROTOBOARDS.

The layout of a typical protoboard is shown in the diagram below with two wires added.

Each hole on the board will accommodate one wire.  The lines show which holes are joined together by the metal springs.  The holes in the main body of the protoboard are joined vertically, with a gap in the middle to accommodate integrated circuits.  The holes at the top and bottom of the board are joined horizontally and can be used as power supply lines.

1).	Add another wire to the diagram above which will connect to wire 1.  Label this with a 1.

Add another wire to the diagram above which will connect to wire 2.  Label this with a 2.

2).	Add two more wires to the diagram above that are connected by the protoboard.  Label these 	with the letter P

Basic Safety.

This task introduces the basic safety requirements for Electronics.

Failure to adhere to them is likely to result in exclusion from the course.

This task is to give an introduction to Safety.

By the end of this task you will be able to:-

Recognise potential safety hazards,

Know how to minimise safety hazards,

Carry out a Risk Assessment.

There are four fundamental dangers associated with Electronics.  Summarise the dangers below:-

1).	Electric shock:-

Mains, ...................................................................................................................................

...................................................................................................................................

Voltages stored in capacitors, ................................................................................................

....................................................................................................................................

Induced voltages from inductors, ...........................................................................................

.....................................................................................................................................

Liquids, ...................................................................................................................................

.....................................................................................................................................

2).	Burns:-

From electrocution, .................................................................................................................

.....................................................................................................................................

From hot components, .............................................................................................................

.....................................................................................................................................

From hot cutting tools, ............................................................................................................

.....................................................................................................................................

From soldering, .......................................................................................................................

.....................................................................................................................................

3).	Cuts:-

Sharp objects, ..........................................................................................................................

.....................................................................................................................................

Cutting tools, ............................................................................................................................

.....................................................................................................................................

4).	Flying debris:-

Sharp cutting tools, ..................................................................................................................

.....................................................................................................................................

Overloaded devices, .................................................................................................................

.....................................................................................................................................

Trip hazards, .............................................................................................................................

.....................................................................................................................................

RISK ASSESSMENTS

A Risk Assessment should be carried out for each activity undertaken.

The dangers should be assessed and a procedure put in place to eliminate the danger.

If the danger cannot be eliminated then it methods should be adopted to ensure that the danger is minimised by either using Personal Protective Equipment, or by adopting or changing the procedure.

1).	Identify the risks:-....................................................................................................................

..................................................................................................................................................

2).	Procedure to eliminate the risks:-.............................................................................................

..................................................................................................................................................

3).	Procedure to minimise the risks:-.............................................................................................

..................................................................................................................................................

4).	Personal Protective Equipment needed:-..................................................................................

..................................................................................................................................................

Light Emitting Diode or LED.

This task is to give an introduction to LEDs, which can be used as Output Subsystems.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Recognise a LED,

Know how to use a LED.

A LED is a small semiconductor component that will emit light when an electric current passes.

1).	Examine a LED and draw a sketch.  Remember to show one lead longer than the other and 	also the small flat part on the one side of the body.

2).	Draw the symbol for a LED.

3).	How many volts are needed across a LED to make it light?

……………………………………………………………………………………………..

4).	How much current should flow through a LED?

……………………………………………………………………………………………..

5).	State three uses of LEDs.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

6).	What are the advantages of LEDs compared to normal lamps?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

Resistors.

This task is to give an introduction to Resistors.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Recognise a resistor,

Know how to work out its value,

A resistor is a component that reduces the amount of electric current that flows.

1).	Examine a selection of resistors and draw sketches to show what they are like.

2).	Draw a labelled diagram to show the cross section of a resistor.

3).	All electronic components are represented by symbols since they are easier to draw than 	pictures.  Draw the symbol that is used to represent a resistor.

4).	What is the unit of resistance?

……………………………………………………………………………………………..

5).	(a)	Construct a Resistor Colour Code Calculator.

(b)	What is the resistance of a resistor with the following colours:-

(i)	Yellow, Purple Brown, Gold

……………………………………………………………………………………………..

(ii)	Brown, Black, Green, Gold

……………………………………………………………………………………………..

(ii)	Brown, Black, Orange, Gold

……………………………………………………………………………………………..

6).	Explain what is meant by a Preferred Value.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

Homework Assignment.

Complete the Homework Exercise “Resistor Colour Codes 1”.  Print out and hand in the Score sheet.

Advanced work.

Use the Support booklet to answer the following questions.

7).	Explain what is meant by the Tolerance of a resistor.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

8).	Name three types of resistors and describe their differences, advantages and disadvantages.

(i) ……………………………………………………………………………..…………..

……………………………………………………………………………………………..

(ii) ……………………………………………………………………..…………………..

……………………………………………………………..……………..………………..

(iii) …………………………………………………………………………..……………..

………………………………………………………………………………..……………..

Practical Work.

1).	The diagram of the circuit that you are going to construct is shown 	opposite.  Label the components.

2).	Collect a protoboard, LED and a correctly coloured resistor.

Decide how you are going to arrange them on your protoboard to 	construct the circuit.

DO NOT pull the leads of the LED too far apart, or they will break!

Use a battery connector to connect your circuit to a 9V battery.

Ensure that your circuit is correct before connecting it to the 9 volt 	power supply.

Mark onto the diagram of the protoboard where you have inserted the components.

3).	If all is well the LED should light.  If it does not light check your work.

4).	What happens if the power supply is connected the other way round to your circuit?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

5).	Explain the purpose of the resistor.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

A Basic Water Alarm System.

This task is to give an introduction to a basic electronics system.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will:-

Be able to recognise an IC,

have constructed a working electronic system,

know one input, processor and output subsystem.

This assignment introduces Integrated Circuits, (ICs).

An IC consists of a tiny piece of silicon onto which complex circuits are fabricated directly.

Putting circuits into IC form has many advantages, the main one for the user is that very complex circuits can be constructed quickly by using ICs as building blocks.

The IC used in this assignment is a digital circuit.

It contains four separate NAND gate circuits and it is one of these that will be used as the processor for the Water Alarm.

The serial number of the IC is 4093  (or 4011).

Draw the symbol for a NAND gate opposite

And label the inputs and outputs.

The IC has 14 pins for connections.

Complete the diagram below to the connections to the pins of the IC looking down from the top.

Each of the NAND gates of the IC are identical, so any can be used for the following circuits.

However, the NAND gate inputs are easily damaged by static electricity and so ALL of the unused inputs SHOULD be connected to 0V connection to the battery.

For the circuits that follow, the IC should be carefully placed across the centre of your protoboard, as in the diagram below.

The NAND gate is an ACTIVE component (i.e. it can amplify) as will function as the processor in the basic Water Level Alarm system.

It is customary not to mark the power supply connections to the IC on the circuit diagram but to assume that they are connected.

DO NOT FORGET TO MAKE THESE CONNECTIONS

WHEN CONSTRUCTING YOUR CIRCUITS

Label the components in the circuit diagram below, including the pin numbers of the IC that you are going to use.

1).	R1 is a 1M, 5% resistor.  Write down the coloured bands that the resistor will have.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

2).	R2 is a 470, 5% resistor.  Write down the coloured bands that the resistor will have.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

3).	Draw on the protoboard diagram below (in pencil) an arrangement for how the components can be arranged to construct the circuit.

Remember to make the power supply connections.

4).	Collect the components and carefully construct the circuit on a protoboard, following your 	design on the planning sheet.  Use single core wire for making other connections on the 	protoboard.

Check the design with your supervisor before continuing.

5).	Connect a battery connector to the circuit.

Ensure that the positive wire (RED) is connected to the positive terminal and the negative 	wire (BLACK) to the negative terminal.

Connect the battery.  What happens?

……………………………………………………………………………………………..

6).	Connect another piece of wire approximately 20 cm long to the positive line and touch the 	free end onto the free end of the INPUT wire.

Connect the battery.  What happens this time?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

7).	Fetch a beaker of water and touch the free ends of the two wires into the water.

Explain what happens this time.  Use the support booklet to help you.

What is the NAND gate acting as?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

8).	Complete the system diagram for the Basic Water Alarm, by labelling the subsystems.

Advanced work.

Use the Support booklet to help you answer the following questions.

8).	Wet your fingers and hold the free ends of the two wires.  Explain what happens.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

9).	Explain how this circuit could be used as a water level alarm to prevent a bath being 	overfilled.

How could you improve this basic circuit?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

How to Solder.

This task is to give an introduction to soldering.

By the end of this task you will:-

Be aware what solder is,

Be aware of how to solder,

Be aware the health and safety and environmental issues of soldering.

Read through the paragraphs below and then answer the questions.

Successful soldering

Soldering is a technique that is employed in electronics to provide reliable, low resistance connections between wires and components.  Solder is not intended to be a type of glue or a filler (although it will actually do these jobs quite successfully.)

Solder is an alloy (mixture of two or more metals). Historically solder was made from tin and lead in the ratio of 3:2. and melted at a temperature of approximately 185oC.

However, because of the environmental problems caused by the lead in the solder when redundant electronic equipment is thrown away in land fill sites, lead free solder is now used on most commercial electronic equipment.  This consists of an alloy of tin, silver and copper and has a melting point of approximately 225°C

Solder will make good connections to copper, brass, tin, silver and gold.

The solder used for electronic work comes as soldering wire in two main sizes, 18 SWG or 22 SWG.  As well as the solder it also contains a flux which helps the solder to flow more readily onto the metal surfaces.  A diagram showing the cross section of solder is shown in figure opposite.  The flux melts along with the solder and appears as a 'smoke'.  This 'smoke' should not be inhaled since it can cause respiratory irritations.

For metals to be soldered they must be clean and free from any grease or corrosion.  A soldering iron suitable for modern electronics work will have a power rating of 15W and a small 'bit' with a diameter of 2mm to 4mm.  The bit is made from copper and is coated with tin.  It will reach a temperature of up to 350oC and, in normal use, will last a long time.  It will become coated with black material (often copper oxide or burnt grease) and should be cleaned by being wiped with a damp sponge.

The tip of the bit of the soldering iron should be kept coated with a thin layer of solder and so after it has been cleaned it should be touched onto the soldering wire to coat it.  This process is known as tinning.

For two metals to be joined by solder it is important to ensure that both metals and the solder are at a temperature above which the solder melts.

Soldering wires to terminal connections

The wire is passed through the hole in the terminal.  It should not be wrapped around the terminal unless you are absolutely certain that you will never want to remove it again!

The bit of the soldering iron should be placed against the terminal and the wire until they are hot enough to melt solder (two or three seconds) and then solder should be applied.  If all is at the right temperature then the solder will be seen to melt and flow onto the wire and terminal.  Sufficient solder should be used to completely cover the terminal hole and the wire where it passes through the terminal.  When this has happened, the solder wire should be removed and then the soldering iron should be removed.  It will take a few seconds for the metals to cool sufficiently for the solder to freeze, but it could remain hot enough to cause burns for several minutes afterwards.

Soldering wires to circuit boards

A similar process is used to that for soldering to terminal

connections.  The wire is passed through the hole in the circuit board.  The bit of the soldering iron is placed against the circuit board copper and the wire for one or two seconds and then solder is applied to the bit of the iron.  If all is at the right temperature then the solder will be seen to melt and flow onto the wire and the circuit board copper.  Sufficient solder should be used to completely cover the circuit board hole and the wire where it passes through the hole.  When this has happened, the soldering wire should be removed and then the soldering iron should be removed.  It will take a few seconds for the metals to cool sufficiently for the solder to freeze, but it could remain hot enough to cause burns for several minutes afterwards.

Common soldering faults

1).	Symptom:	The wire is coated with solder but the circuit board copper is not.

Cause:		The soldering iron did not warm the copper of the circuit board 					sufficiently.

Remedy:	Reheat the copper board and the wire and apply some more solder.

2).	Symptom:	The circuit board copper is coated with solder but the wire is not.

Cause:		The soldering iron did not warm the wire sufficiently.

Remedy:	Reheat the circuit board copper and the wire and apply some more 					solder.

3).	Symptom:	The wire appears to be soldered correctly but it has formed a high 					resistance joint.

Cause:		This is known as a DRY JOINT and is caused either by dirt on the 				wire or the wire not being heated sufficiently.

Remedy:	Reheat the copper board and the wire and apply some more solder.

If the joint still has a high resistance then the wire should be removed 				and cleaned before being re soldered.

The timing for soldering components, especially semiconductors, is very important.  If the wire and circuit board are not adequately heated then a poor joint will result.  If the wire and circuit board are heated too much then there is a great danger of damaging the component, especially when using lead free solder.

Semiconductor devices will usually withstand the heat of a soldering iron for about 2 seconds.  Quick, careful and effective soldering is vital.  If you are uncertain how quickly you will be able to solder a delicate (or expensive) component, a Heat Shunt should be used to conduct the heat away from the component.  A pair of small nose pliers will act as a convenient heat shunt for components with leads, e.g. transistors.  The lead being soldered is held with the pliers.  Excessive heat is then conducted to the pliers instead of the transistor.

Solder should NEVER be applied to the bit of the soldering iron and then transported to the wire and circuit board copper.  At the best, the solder joint will be of very poor quality because the flux needed to help the solder flow will have burnt off from the soldering iron while the molten solder was being transported.  Solder should only be applied to the hot wire and copper when they have been heated by the soldering iron.

Answer the following questions.

1).	What is solder made from?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

2).	At what temperature does solder melt?

……………………………………………………………………………………………..

3).	Name three metals that can be soldered.

……………………………………………………………………………………………..

4).	Suggest a reason why the surfaces to be soldered must be cleaned if they are greasy or 	corroded.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

5).	How should the soldering iron bit be cleaned if it is dirty?

Why should you never file the soldering iron bit to clean it?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

6).	Explain, in your own words, how to solder a wire onto a piece of copper.  Draw diagrams to 	help if you wish.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

7).	Suggest a reason why it is necessary to ensure that solder has flowed onto both of the 	surfaces to be joined?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

8).	What happens to the components being soldered if heat is applied to them for too long?

……………………………………………………………………………………………..

9).	What is a HEAT SHUNT and how is it used?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

10).	What components are most likely to suffer damage from excess heat?

……………………………………………………………………………………………..

11).	Why should work that has been freshly soldered not be touched.

……………………………………………………………………………………………..

12).	Explain briefly why wearing goggles or safety glasses is a sensible safety precaution when 	soldering.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

13).	Explain briefly what you should do if you burn yourself with a soldering iron?

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

14).	Explain why hot soldering irons should not be left lying on a bench but should be returned 	to a soldering iron stand.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

15).	Explain why the electronics industry has switched to Lead Free solder.

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

……………………………………………………………………………………………..

Stripboard and the Soldering Test.

STRIPBOARD.

While protoboards provide a very quick means of assembling and testing circuits, they do not provide a reliable circuit for normal use.  Printed circuit boards provide the most reliable means of ruggedly constructing circuits but the design of the circuit board can take a long time and is not easily modified.  A more convenient way to produce rugged and reliable circuits is to use stripboard.  The diagram below shows a piece of stripboard viewed from the bottom.

It consists of a sheet of SRBP board punched with holes every 0.1 inch and with strips of copper foil running horizontally.  These copper foil tracks are used to make connections between the components by soldering the components to the copper tracks.

In use, the components would be placed on the TOP side of the board with the wires poked through the holes.  The wires are then soldered to the copper track and the excess wire removed.

THE SOLDERING TEST.

Since the strips on stripboard are only 0.1 inches apart it is very easy to spread solder from one strip to another.  This causes short circuits and is of little use.  This test is to give you practice at soldering to strip board.

Obtain an offcut of strip board and solder six wires to adjacent copper strips along the edge of the board.  Seek assistance from your supervisor if you have difficulty with this task.

When completed have your work assessed by your supervisor and attach the sample to your Induction Booklet.

Supervisor Signature:.........................................................................................................................

Date:...................................................................................................................................................

Homework Assignment.

Complete the Online True/False “Induction Test”.  Print out and hand in the Score sheet.

Make sure that you have also completed the Online True/False “Initial Test”.