NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

INPUT

SUBSYSTEMS

PACKAGE

(1).	Light Dependent Resistor  (LDR)

(2)	The Night Light

(3).	NTC Thermistor

(4)	The Refrigerator Alarm

(5).	Switches

Light Dependent Resistor  LDR

This task is to give an introduction to LDRs, which can be used as Input Subsystems.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Recognise a LDR,

Know how that its resistance decreases with increasing light intensity.

Estimate the resistance of an LDR at a given light intensity from its characteristic.

Estimate the light intensity needed for a LDR to have a given resistance from its characteristic.

Use with a processor to form a light sensing system.

LDRs are light sensitive devices whose resistance varies considerably with changing light intensity.

1).	(a)	Examine a LDR and draw a diagram of it in the space below.

(b)	Draw the symbol for a LDR in the space below.

2).	Use a digital meter, set to measure resistance, and measure the resistance of a LDR when 	exposed to light and also when in the dark.

(a)	Resistance in the dark................................................................................................

(b)	Resistance in the light................................................................................................

3).	The range over which the resistance of an LDR changes is considerable.

To show such a large range on a graph it is necessary to use graph paper with a
	logarithmic scale.

A graph of light intensity against resistance for a typical LDR is shown on the next page.

Use the graph to answer the following questions.

(a)	What is the resistance of the LDR at illumination levels of:-

(i)	100 lux.........................................................................................................................

(ii)	2000 lux.......................................................................................................................

(iii)	60 klux.........................................................................................................................

(iv)	300 klux.......................................................................................................................

(v)	40 lux..........................................................................................................................

(b)	What level of illumination corresponds to a LDR resistance of:-

(i)	300.............................................................................................................................

(ii)	2.5k............................................................................................................................

(iii)	8k...............................................................................................................................

(iv)	20k.............................................................................................................................

(v)	60k.............................................................................................................................

Homework Assignment.

Complete the Homework Exercise “LDR Graph”.

Print out and hand in the Score sheet.

4).	The Night Light

An electronic system is needed which will automatically turn on a light when it gets dark

Such a system can be made from the Basic Water Alarm system from the Induction Package 	with just a few changes as shown in the circuit diagram below.

(a)	The 1MΩ resistor is changed to a 10kΩ resistor.

What will be the colour code for this resistor?

...............................................................................................................................................

(b)	The LDR is connected to the input of the circuit.

(c)	Plan your circuit onto the diagram below.  You can base it on your Water Alarm.

(d)	Construct your circuit and connect the battery the correct way round.

(i)	When your circuit is in the light, what is the state of the LED?

...............................................................................................................................................

(ii)	When your circuit is in the dark, what is the state of the LED?

...............................................................................................................................................

(e)	How effective is your circuit as a ‘Night Light’?

...............................................................................................................................................

...............................................................................................................................................

(f)	There are three ways in which the circuit can be modified to work more effectively.  		The easiest is to swap the 10kΩ resistor and the LDR as shown in the circuit diagram 		below.

Modify your circuit.  What happens when the LDR is in the dark this time?

...............................................................................................................................................

...............................................................................................................................................

(g)	Suggest two other ways in which the original circuit could be modified to make the 		circuit work correctly as a ‘Night Light’.

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

(h)	Connect a voltmeter across the LDR.

Vary the amount of light reaching the LDR by covering it with your hand and record 		below the voltage at which the LED turns on and also turns off.

Turns On:-.............................................................................................................................

Turns Off:-............................................................................................................................

What do you notice about these voltages?

...............................................................................................................................................

...............................................................................................................................................

(i)	Replace the 10kΩ resistor with a 4.7kΩ resistor.

(i)	What are the coloured bands for the 4.7kΩ resistor?

...............................................................................................................................................

(ii)	Describe the effects that this change has on your circuit.

Does this alter the voltage readings at which the LED switches On and Off?

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

Advanced Work

(j)	Explain how you could modify the circuit to enable the sensitivity of the circuit to be 		changed.

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

Thermistor (ntc)

This task is to give an introduction to Thermistors, which can be used as Input Subsystems.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Recognise a thermistor,

Know how that its resistance decreases with increasing temperature.

Estimate the resistance of a thermistor at a given temperature from its characteristic.

Estimate the temperature needed for a thermistor to have a given resistance from its characteristic.

Use with a processor to form a temperature sensing system.

Thermistors are heat sensitive devices whose resistance varies considerably with changing temperature.

1).	(a)	Examine a thermistor and draw a diagram of it in the space below.

(b)	Draw the symbol for a thermistor in the space below.

2).	Use a digital meter, set to measure resistance, and measure the resistance of a thermistor 	at 	room temperature, and then warmed with your hands.

(a)	Resistance at room temperature ................................................................................

(b)	Resistance when warmed ..........................................................................................

3).	(a)	How does the resistance vary with temperature?

...............................................................................................................................................

...............................................................................................................................................

(b)	What would you expect to happen to the resistance of the thermistor if it were 			cooled?

...............................................................................................................................................

4).	The range over which the resistance of a thermistor changes is considerable.

To show such a large range on a graph it is necessary to use graph paper with a
	logarithmic scale.

A graph of temperature against resistance for a typical thermistor is shown below.

Use the graph to answer the following questions.

NOTE that this graph has a linear scale for temperature and a logarithmic scale for 	resistance.

(a)	What is the resistance of the thermistor at temperatures of:-

(i)	25°C.......................................................................................................................

(ii)	50°C.......................................................................................................................

(iii)	10°C.......................................................................................................................

(iv)	15°C........................................................................................................................

(v)	45°C........................................................................................................................

(b)	What temperature corresponds to a thermistor resistance of:-

(i)	300k...........................................................................................................................

(ii)	2.5k............................................................................................................................

(iii)	8k...............................................................................................................................

(iv)	20k.............................................................................................................................

(v)	60k.............................................................................................................................

4).	The Refrigerator Alarm

An electronic system is needed which will automatically produce a warning if a refrigerator 	becomes too warm.

Such a system can be made from the Night Light system from the with just a few changes as 	shown in the circuit diagram below.

(a)	What will be the colour code for the 100kΩ resistor?

...............................................................................................................................................

(b)	The Thermistor is connected to the input of the circuit.

(c)	Plan your circuit onto the diagram below.  You can base it on your Night Light.

(d)	Construct your circuit and connect the battery the correct way round.

(i)	When your thermistor is warm, what is the state of the LED?

...............................................................................................................................................

(ii)	When your thermistor is cold, what is the state of the LED?

...............................................................................................................................................

(e)	How could you adjust the temperature at which the LED lights?

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

(f)	How would you modify your system to make an effective alarm system for a 			freezer?

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Thermistor Graph”.

Print out and hand in the Score sheet.

Switches

This task is to give an introduction to Switches, which can be used as Input Subsystems.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Recognise some of the different types of switches,

Know that when closed a switch has a low resistance,

Know that when open a switch has a very high resistance,

Use with a processor to form a sensing system.

The switch is an extreme example of a resistive input transducer since it has either a very high resistance or a very low resistance.

(a)	There are many different types of switch, a selection of which are listed below.

Use component catalogues to write a description of each type of switch.

Draw diagrams to help your descriptions.

State one use for each typ of switch.

(i)	Single Pole Single Throw (SPST) Toggle switch

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(ii)	Double Pole Double Throw (DPDT) Toggle switch

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(iii)	Normally Open (NO) push switch

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(iv)	Normally Closed (NC) push switch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(v)	DPDT slide switch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(vi)	4 pole 3 way rotary switch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(vii)	1 pole 12 way rotary switch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(viii)	SPDT microswitch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(viii)	A reed switch

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(viii)	A tilt switch.

...................................................................................................................

...................................................................................................................

...................................................................................................................

Use:-  .........................................................................................................

(b)	How you would use a switch as a sensor subsystem?

...............................................................................................................................................

...............................................................................................................................................

(c)	How would you connect a NO push switch to the Basic Water Alarm, so that it turned off 	the LED when the switch was pressed?

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

Homework Assignment.

Complete the Online True/False “Input Subsystems Test”.  Print out and hand in the Score sheet.