NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

BASIC

ELECTRICITY

AND

OHM'S LAW

PACKAGE

(1).	The Nature of Electricity.

(2).	Ohm's Law.

(3).	Ohm's Law calculations.

(4).	Combinations of resistors.

(5).	Electrical Power.

The Nature of Electricity.

The aim of this assignment is to develop an understanding of what electricity is.

Use the Support Booklet and any practical equipment that you need to carry out the following tasks.

By the end of this task you will be able to:-

Appreciate that a simple model of electricity consists of a flow of electrons,

Appreciate how how this relates to an electric current and voltage.

Know how to measure a current and a voltage.

Read through the passage below and, using the Support Booklet, answer the following questions.

Electricity is the result of the movement of very small quantities of electric charge called ELECTRONS.  When electrons move, an electric current is produced which is measured in AMPs by an AMMETER.

To measure a current the ammeter has to be connected so that the current has to flow THROUGH the ammeter.  It has to be connected in SERIES.

The energy that electrons give out when going through a circuit is measured by their VOLTAGE (potential difference, p.d.) in VOLTs.  A VOLTMETER is the instrument used to measure voltage and is connected ACROSS the circuit.  It has to be connected in PARALLEL.

An electric current can only flow around a circuit when the electrons have energy, i.e., when there is a voltage.

Electrons are never LOST in a circuit.  All of the electrons that start to flow around a circuit from a battery end up entering the other side of the battery.  Electrons do loose energy by the work that they do in going through the various parts of the circuit.

QUESTIONS.

a).	(i)	Explain what is meant by an electric current.

................................................................................................................................................

................................................................................................................................................

(ii)	State the units of electric current.

................................................................................................................................................

b).	What causes a current to flow around a circuit?

................................................................................................................................................

c).	(i)	Explain how you would measure an electric current.

................................................................................................................................................

................................................................................................................................................

(ii)	Draw a circuit diagram to show how you would connect an ammeter to 				measure the current flowing through a lamp from a battery.

d).	(i)	Explain what is meant by a voltage.

................................................................................................................................................

................................................................................................................................................

(ii)	What units is voltage measured in?

................................................................................................................................................

e).	(i)	Explain how you would measure a voltage.

................................................................................................................................................

................................................................................................................................................

(ii)	Draw a circuit diagram to show how you would connect a voltmeter to 				measure the voltage across a lamp.

Ohm's Law.

The aim of this assignment is to investigate how the current flowing through a piece of wire varies with the voltage across it.  The law that results from this investigation is a very important, and underpins ALL electronics calculations.

By the end of this assignment you will be able to use Ohm’s Law to calculate resistance, current and voltage for basic circuits.

You will need the following equipment:-

1x	unregulated 12V power supply,

1x	1A ammeter,

1x	10V voltmeter,

1x	11 ohm rheostat,

1x	15 ohm wire wound resistor,

7x	connecting leads.

Use the equipment to make up the circuit shown in the diagram opposite.

BEFORE switching on, check with your supervisor that it is connected correctly.

Adjust the rheostat until the voltmeter reads each of the values in turn shown in the table and note down the reading on the ammeter.

| Voltage (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
| 0 |  |  |
| 1.0 |  |  |
| 2.0 |  |  |
| 3.0 |  |  |
| 4.0 |  |  |
| 5.0 |  |  |
| 6.0 |  |  |
| 7.0 |  |  |
| 8.0 |  |  |
| 9.0 |  |  |
| 10.0 |  |  |

Divide the voltage by the current for each set of values in the table and record the answers in the last column of the table.

Plot a graph of your results on page 5, with voltage on the vertical (y) axis and current on the horizontal (x) axis.  Draw the best straight line that will go through the points that you have plotted.

Find the gradient of your graph and record your answer on the graph paper.

What do you notice about the gradient of your graph and the values in the last column of the table?

................................................................................................................................................

................................................................................................................................................

CONCLUSION.

The graph is a straight line passing through the origin and shows that the current through the resistor is proportional to the voltage across it, (so long as the temperature remains constant.)

Ohm's Law calculations.

Ohm's law can be placed into a Magic Triangle

to help with its re-arrangement.

The magic triangle is shown opposite.

To find a formula, cover up the letter that you require and the formula can be read from the triangle.

eg, to find I, cover the letter I and then I = V/R.

Questions.

These are to provide practice at using Ohm's Law.

A	Calculate the resistance of each resistor.

(1).	V = 6V	I = 2A		R =

(2).	V = 12V	I = 3A		R =

(3).	V = 2V	I = 4A		R =

(4).	V = 7V	I = 3.5A	R =

(5).	V = 8V	I = 1A		R =

(6).	V = 1V	I = 10A	R =

(7).	V = 9.2V	I = 0.4A	R =

(8).	V = 1000V	I = 2.5A	R =

(9).	V = 25V	I = 0.05A	R =

(10).	V = 0.01V	I = 10A	R =

B.	Calculate the voltage V across each resistor.

(11).	I = 1A		R = 7W	V =

(12).	I = 3A		R = 4W	V =

(13).	I = 2A		R = 19W	V =

(14).	I = 2A		R = 4W	V =

(15).	I = 15A	R = 5W	V =

(16).	I = 2.5A	R = 10W	V =

(17).	I = 1.1A	R = 3W	V =

(18).	I = 4.7A	R = 2.1W	V =

(19).	I = 13.2A	R = 4.7W	V =

(20).	I = 0.005A	R = 3.4W	V =

C.	Calculate the current through each resistor.

(21).	V = 4V	R = 2W	I =

(22).	V = 20V	R = 2W	I =

(23).	V = 15V	R = 7.5W	I =

(24).	V = 10V	R = 20W	I =

(25).	V = 4V	R = 4W	I =

(26).	V = 2.2V	R = 1.1W	I =

(27).	V = 9.6V	R = 3.2W	I =

(28).	V = 0.07V	R = 0.28W	I =

(29).	V = 0.1V	R = 0.001W	I =

(30).	V = 0.06V	R = 0.6W	I =

More Ohm's Law calculations.

Complete the missing values in the table below.

| VOLTAGE (V) | CURRENT (A) | RESISTANCE () |
| --- | --- | --- |
|  | 2 | 5 |
| 2.5 | 0.5 |  |
| 240 |  | 960 |
|  | 13.2 | 4.7 |
| 25.0 | 0.05 |  |
| 9.2 |  | 0.4 |
| 3.3 | 0.16 |  |
|  | 0.000023 | 56000 |
| 12.0 |  | 680 |
|  | 1.3x10-5 | 82000 |

The circuit shown below is connected and the meters give the readings shown.

a).	What is the battery voltage?

................................................................................................................................................

b).	What is the total current flowing from the battery?

................................................................................................................................................

c).	What is the total resistance of the circuit?

................................................................................................................................................

d).	What is the resistance of R2?

................................................................................................................................................

................................................................................................................................................

e).	What current flows through the lamp?

................................................................................................................................................

f).	What is the voltage across the lamp?

................................................................................................................................................

g).	What is the resistance of the lamp?

................................................................................................................................................

................................................................................................................................................

h).	What is the resistance of R1?

................................................................................................................................................

................................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Ohm’s Law”.

There are two options for this exercise, Level 2 (GCSE) and Level 3 (AS).

Choose the most appropriate level and then print out and hand in your Score sheet.

Combinations of resistors.

The aim of this assignment is to learn and provide practice at calculating resistor combinations.  You may find the Support booklet helpful.

By the end of this assignment you will be able to calculate basic combinations of resistors.

Resistors in Series

If two or more resistors in a circuit are connected in series, their combined resistance can be found simply by adding together their separate resistances.

1).	Draw a circuit diagram of two resistors in series.  Label the resistors R1 and R2 and write, 	mathematically, how you would find the total resistance.

................................................................................................................................................

2).	Find the combined resistance RT of the following resistors when connected in series:-

a).	5W	and	10W		RT =

b).	100W	and	220W		RT =

c).	500W	and	1kW		RT =

d).	4.7kW	and	2.7kW		RT =

e).	8.2kW	and	3.9kW		RT =

Connecting resistors in series increases the total resistance because, when a current passes through the combination, each resistor opposes the flow of electrons.

Homework Assignment.

Complete the Homework Exercise “Resistor Combinations”.

Choose Option 1, “Resistors in Series”

Print out and hand in your Score sheet.

Resistors in Parallel

In a parallel circuit, there is more than one current path.  It is therefore easier for the current to pass, so the total resistance is less.

3).	Use the Support booklet to help you draw a diagram to show two resistors connected in 	parallel.  Label the resistors R1 and R2 and write, mathematically, how you would find the 	total resistance of the two resistors in parallel.

................................................................................................................................................

4).	Find the total resistance of each of these parallel pairs:-

a).	4W	and	12W		RT =

................................................................................................................................................

b).	50W	and	200W		RT =

................................................................................................................................................

c).	220W	and	220W		RT =

................................................................................................................................................

d).	4.7kW	and	2.7kW		RT =

................................................................................................................................................

e).	56kW	and	68kW		RT =

................................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Resistor Combinations”.

Choose Option 2, “Resistors in Parallel”

Print out and hand in your Score sheet.

Combinations of resistors in Series and Parallel

1).	A 100W resistor is connected in parallel with a series combination of two 25W resistors.

(a)	Draw a diagram to show how the resistors are connected.

b).	Calculate the total resistance of the combination.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

c).	If a 12V battery is connected across the combination, what current will flow?

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

More combinations of resistors.

Find the total resistance in each case, i.e. find the value of the single resistor which would effectively replace each network of resistors.

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

Addition Exercises

There are two more Homework Exercise “Resistor Combinations” options.

Option 3 provides series and parallel resistor combinations, while Option 4 provides a mixture of questions.

You should use these other options to practice calculationg the total resistance of combinations.

Electrical power.

The aim of this assignment is to learn and provide practice at electrical power calculations.

By the end of this assignment you will be able to:-

Know that electrical power is the amount of electrical energy used per second.

Carry out basic calculations on electrical power.

Relate electrical power to the resistance of the circuit.

Determine appropriate fuse ratings for circuits.

The electrical power consumed in a circuit or component is found by using the following formula:-

Power = Voltage x Current	(W = V x I)

The unit of power is the WATT or Joule per second.

1).	Put the above formula into a magic triangle.

2).	If a car headlamp has a current flowing of 5A when connected to 12V, what power is being 	used?

................................................................................................................................................

................................................................................................................................................

3).	What is the power rating of a soldering iron which has a current of 2A when connected to a 	24V supply.?

................................................................................................................................................

................................................................................................................................................

4)	a).	What current passes through a 1000W electric fire operating from 230V mains?

................................................................................................................................................

................................................................................................................................................

b).	What is the resistance of the electric fire element?

................................................................................................................................................

................................................................................................................................................

c).	What value fuse would you use in the plug of the electric fire?

................................................................................................................................................

5)	a).	What current passes through a 60W lamp operating from 230V mains?

................................................................................................................................................

b).	What is the resistance of the filament of the lamp?

................................................................................................................................................

................................................................................................................................................

6).	A voltage of 22V is connected across a 220W resistor.

a).	What current passes through the resistor?

................................................................................................................................................

................................................................................................................................................

b).	What is the power used by the resistor?

................................................................................................................................................

................................................................................................................................................

7).	Calculate the current passing when each of the following is connected to a 230V mains 	supply and state the required fuse for the mains plugs:-

a).	a 2kW kettle,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

b).	a 60W lamp,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

c).	a 250W television,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

d).	a 700W microwave cooker.

Current ...................................................................................................................................

Fuse ........................................................................................................................................

8).	A LED is to be operated from a 12V supply, by connecting it in series with a resistor.

a).	Draw a circuit diagram of a LED connected in series with a resistor.

b).	If the LED needs a voltage of 2V across it, what is the voltage across the series 			resistor when connected to the 12V supply?

...............................................................................................................................................

...............................................................................................................................................

c).	If a current of 25mA passes through the LED, what current passes through the 			resistor?

...............................................................................................................................................

...............................................................................................................................................

d).	Calculate the required value of the series resistor.

...............................................................................................................................................

...............................................................................................................................................

e).	Calculate the power dissipated in the series resistor.

...............................................................................................................................................

...............................................................................................................................................

f).	Calculate the power dissipated in the LED.

...............................................................................................................................................

...............................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Power Calculations”.

There are two options for this exercise, Level 2 (GCSE) and Level 3 (AS).

Choose the most appropriate level and then print out and hand in your Score sheet.

Complete the Online True/False “Ohm’s Law Test”.

Print out and hand in the Score sheet.