NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

OP-amp

AMPLIFIER

PACKAGE

(1).	Operational Amplifiers.

(2)	Inverting Amplifier

(3).	Non-inverting Amplifier

(4).	Negative and Positive Feedback

(5)	Summing Amplifier

(6)	Difference Amplifier

(1).	The Operational Amplifier.

The aim of this assignment is to revise the operational amplifier, op-amp, and to become familiar with its applications.  You will need to use the Support Booklet.

1).	(a)	(i)	Draw a diagram to show the symbol for an op-amp and label the inverting 				input, the non-inverting input, the output and the power supply connections.

(a)	(ii)	Write down four general properties of op-amps.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	(i)	The op-amp amplifies the voltage difference between the two input terminals.

What is the formula that relates the input voltages to the output voltage?

..................................................................................................................................

..................................................................................................................................

(b)	(ii)	Explain what limits the maximum output voltage.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Draw a large diagram to show the pin connections of a typical op-amp IC.

Label the two power supply connections, the output, the inverting input and the non-		inverting input terminals.

ALL COMMON OP-AMPS HAVE THESE PIN CONNECTIONS (741, TLO8l, CA3140 etc.).

(d)	(i)	Explain what is meant by the voltage gain of an amplifier.

..................................................................................................................................

..................................................................................................................................

(d)	(ii)	If the input signal to an amplifier is 5mV and the output is 15V,
			what is the voltage gain of the amplifier?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	(iii)	Explain what is meant by the power gain of an amplifier.

..................................................................................................................................

..................................................................................................................................

(d)	(iv)	If the input signal power to an amplifier is 25mW and the output power is 				20mW, calculate the power gain of the amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(e)	(i)	In order to ensure that the circuit is stable, the very large voltage gain of an 			op-amp is usually only achieved at low frequencies.  Find out how the voltage 			gain varies with frequency for an op-amp and then draw a sketch of the 				frequency response below.

(e)	(ii)	Explain what is meant by the bandwidth of an amplifier circuit.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(e)	(iii)	The diagram below shows a graph of power gain against frequency for an 				amplifier.

Mark onto the diagram the bandwidth of the amplifier.

(e)	(iv)	What is the bandwidth of the amplifier?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(2).	The Inverting Amplifier.

The aim of this assignment is to understand the inverting amplifier and to become familiar with its applications.  You will need to use the Support Booklet.

The voltage gain of an op-amp is very large, far too large to be of any use as an amplifier for 	audio systems.  It is not possible to alter the gain of the op-amp IC itself, but the overall gain 	of a circuit containing an op-amp can be altered by the use of negative feedback.  This is 	where part of the output is fed back to the input to cancel out some of the input signal.  The 	simplest circuit that uses this principle is the inverting amplifier.

2).	(a)	(i)	What is meant by the voltage gain of an amplifier?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(ii)	Draw a circuit diagram of an inverting amplifier.

(a)	(iii)	What is the formula for the voltage gain of an inverting amplifier?

..................................................................................................................................

..................................................................................................................................

(a)	(iv)	What will be the voltage gain of an amplifier when Rf = 47kW 
			and R1 = l0kW?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(v)	Draw a component layout on the PROTOBOARD planning sheet below of an 			inverting amplifier.

Set up an inverting amplifier on a protoboard using a 47kW resistor for Rf and a
		l0kW resistor for R1.

Use a power supply of +/–5V and set a signal generator to 1kHz and to give a sine 		wave input of 0.lV amplitude.  Use an oscilloscope to measure the input and output 		amplitude.  Draw a sketch of the oscilloscope traces on the diagram below.

Label clearly the input and output waves.

Y1 sensitivity = .........................................

Y2 sensitivity = .........................................

Timebase speed = .........................................

Amplitude of output = .........................................

Period = .........................................

Frequency = .........................................

Explain why the amplifier is said to be INVERTING.

..................................................................................................................................

(a)	(vi)	Increase the input amplitude to 1V and again draw a sketch of the 					oscilloscope traces on the diagram below.  Label clearly the input and output 			waves.

Explain what has happened to the output waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vii)	Describe what happens to the amplitude of the output waveform as the 				frequency of the input is increased towards 1MHz and beyond.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(3).	The Summing Amplifier.

The aim of this assignment is to understand the summing amplifier and to become familiar with its applications.  You will need to use the Support Booklet.

This amplifier circuit is very similar to that of the inverting amplifier except that it can have 	more than one input signal.  The amplifier sums together the input signals and gives an 	amplified output.

3).	(a)	(i)	Draw a circuit diagram of a summing amplifier with two inputs.

(a)	(ii)	Explain how the circuit is able to sum together the two inputs.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iii)	What is the formula for the output voltage of a summing amplifier if the two 			input resistors are of the same value?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iv)	A summing amplifier has 15kW input resistors and a 33kW feedback resistor.

If the two input voltages are +2.5V and –4.5V calculate the output voltage.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(v)	Describe, briefly, two uses for a summing amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vi)	Draw a component layout on the PROTOBOARD planning sheet below of an 			inverting amplifier.

Set up a summing amplifier on a protoboard using a 100kW resistor for Rf and two
		47kW resistor for the input resistors.  Use a power supply of +/–5V.  Connect a 2V, 		50Hz supply (from a power unit) to the one input and a 1V, 200Hz supply (from a 		signal generator) to the other.  Use an oscilloscope to observe the output amplitude.  		When the trace is stable, draw a sketch of the oscilloscope trace on the diagram 			below.

Explain the waveform that you have drawn.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vii)	Replace the 50Hz supply with a 2V, 200Hz square wave supply (from a 				signal generator).  Increase the frequency of the other generator to 1kHz.  				When the trace is stable, draw a diagram of the oscilloscope trace below.

An important application of a summing amplifier is for Digital to Analogue conversion (DAC).

(b)	(i)	What is Digital to Analogue Conversion?

..................................................................................................................................

..................................................................................................................................

(b)	(ii)	State an application where a DAC would be used.

..................................................................................................................................

(b)	(iii)	Draw a circuit diagram of a summing amplifier configured as a four bit DAC.

(b)	(iv)	If the feedback resistor is 10k and the resistor for D3 is also 10k, what 				would be the value of the other input resistors?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

A logic 1 is 5V and a logic 0 is 0V.  Calculate the output voltage for your circuit if there
	is a binary input of:-

(c)	(i)	1001

..................................................................................................................................

(c)	(ii)	0110

..................................................................................................................................

(c)	(iii)	1101

..................................................................................................................................

(4).	The Non-inverting Amplifier.

The aim of this assignment is to understand the non-inverting amplifier and to become familiar with its applications.  You will need to use the Support Booklet.

This is a different arrangement that is used to reduce the voltage gain of an op-amp circuit.

A sample of the output is still fed back to the inverting input of the op-amp, where it again 	cancels out some of the input signal.  However, with this circuit the input signal is connected 	directly to the non-inverting input terminal, which results in the input resistance (impedance) 	of the circuit being very large, (typically 10 - 100MW).

4).	(a)	(i)	Draw a circuit diagram of a non-inverting amplifier.

(a)	(ii)	What is the formula for the voltage gain of a non-inverting amplifier?

..................................................................................................................................

..................................................................................................................................

(a)	(iii)	What will be the voltage gain of an amplifier when Rf = 47kW
			and R1 = l0kW?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iv)	Draw a component layout on the PROTOBOARD planning sheet below of a

non-inverting amplifier.

Set up a non-inverting amplifier on a protoboard using a 47kW resistor for Rf and a
		l0kW resistor for R1.

Use a power supply of +/–5V and set a signal generator to 1kHz and to give a sine 		wave input of 0.lV amplitude.  Use an oscilloscope to measure the input and output 		amplitude.  Draw a sketch of the oscilloscope traces on the diagram below.

Label clearly the input and output waves.

Y1 sensitivity = .........................................

Y2 sensitivity = .........................................

Timebase speed = .........................................

Amplitude of output = .........................................

Period = .........................................

Frequency = .........................................

Explain why the amplifier is said to be NON-INVERTING.

..................................................................................................................................

(a)	(v)	Increase the input amplitude to 1V and again draw a sketch of the 					oscilloscope traces on the diagram below.

`	Label clearly the input and out put waves.

Explain what has happened to the output waveform.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vi)	Describe what happens to the amplitude of the output waveform as the 				frequency of the input is increased towards 1MHz and beyond.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(5).	The Voltage Follower.

The aim of this assignment is to understand the voltage follower and to become familiar with its applications.  You will need to use the Support Booklet.

This circuit can be considered as a special case of the non-inverting amplifier when Rf is 0Ω 	and R1 is ∞.  This gives the circuit a voltage gain of 1 and a very large input resistance, 	(typically 10 - 100MW).

5).	(a)	(i)	Draw a circuit diagram of a voltage follower based on a non-inverting amplifier.

(a)	(ii)	Show that the voltage gain of a voltage follower is 1?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	(i)	State and explain TWO uses for a voltage follower.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(6).	The Difference Amplifier.

The aim of this assignment is to understand the difference amplifier and to become familiar with its applications.  You will need to use the Support Booklet.

This amplifier circuit is based on the inverting amplifier.

The amplifier subtracts the two input signals and gives an amplified output.

6).	(a)	(i)	Draw a circuit diagram of a difference amplifier with two inputs.

(a)	(ii)	Explain how the circuit is able to subtract the two inputs.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iii)	What is the formula for the output voltage of a difference amplifier if the two 			input resistors are of the same value?

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iv)	A difference amplifier has 15kW input resistors and a 33kW feedback resistor.

If the two input voltages are +2.5V to the non-inverting input and –4.5V to 			the inverting input, calculate the output voltage.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(v)	Describe, briefly, two uses for a difference amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vi)	Draw a component layout on the PROTOBOARD planning sheet below of a 			difference amplifier.

Set up a difference amplifier on a protoboard using a 100kW resistor for Rf and two
		47kW resistor for the input resistors.  Use a power supply of +/–5V.  Connect a 1V, 		200Hz supply (from a signal generator) to the one input and a 1V, 200Hz supply 			(from another signal generator) to the other.  Use an oscilloscope to observe the 			output amplitude.  When the trace is stable, draw a sketch of the oscilloscope trace 		on the diagram below.

Explain the waveform that you have drawn.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(vii)	Reduce the amplitude of the signal going to the non-inverting input.

Describe and explain what happens to the output voltage

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	An important application of a difference amplifier is for the reduction of interference 		on signals.

Using diagrams, explain how this is achieved.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(7).	Feedback.

7).	(a)	(i)	Explain what is meant by the term feedback.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(ii)	Explain what is meant by negative feedback.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iii)	Describe the effects of negative feedback on an amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	(i)	Explain what is meant by positive feedback.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	(ii)	Describe the effects of positive feedback on an amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(b)	(iii)	Describe a situation where you would use positive feedback.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(8).	EXAMINATION TYPE QUESTIONS

1).	(a)	(i)	Draw a circuit diagram of an inverting amplifier with R1 = l0kohm and
			Rf = l50kohm.

(a)	(ii)	Calculate the gain of the amplifier.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(a)	(iii)	How does the output signal of this amplifier differ from the input signal?

..................................................................................................................................

..................................................................................................................................

(b)	If the input voltage is 2mV, what will be the output voltage?

..................................................................................................................................

..................................................................................................................................

(c)	Give one reason why resistors are connected to op-amps to turn them

into more usable amplifiers.

..................................................................................................................................

..................................................................................................................................

(1988)

2).	(a)	A non-inverting amplifier circuit is required for a hi-fi system.

It is to be powered from ±15 volts.

Draw a circuit diagram of a non-inverting amplifier using an op-amp.

(b)	The amplifier is to have a voltage gain of 10 and resistor R1 a value of l5kohms. 			Calculate the value of Rf needed.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	The input signal has a peak voltage of 800mV, calculate the peak voltage of the 			output signal.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	If the input signal is a sine wave of frequency 500Hz, draw a labelled sketch to show 		how both the input and output signals would appear on an oscilloscope.

(e)	If the power supply to the op-amp was reduced to ±5 volts, draw a labelled sketch 			showing the output signal and explain why there is a change in the output.