NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

SEQUENTIAL

LOGIC

PACKAGE

(1).	NAND Gate Bistable Latch

(2).	D-Type Flip-Flop

(3)	Shift Register

(1).	The NAND Gate Bistable Latch.

The aim of this assignment is to understand the NAND gate bistable latch and to become familiar with its applications.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a NAND bistable latch,

Explain how a NAND gate bistable latch works.

Know some applications of a NAND gate bistable latch.

You will need to use the Support Booklet.  The NAND gates will be contained in a 4011/4093 IC.

1).	(a)	Draw a diagram to show the pin-out of the 4011/4093 IC.

(b)	Use the support booklet to draw the circuit diagram of a NAND gate bistable latch.  		Use 100k pull up resistors for the inputs and include a LED with a 1k series 			resistor from each output to 0V.

(c)	Use the support booklet to explain how the circuit works.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

2).	(a)	Use the protoboard diagram below to show how you can construct a NAND gate 			bistable latch.  Remember to connect all unused inputs to 0V.

(b)	Construct the circuit and check that it functions correctly by momentarily connecting 		each of the bistable inputs to 0V in turn.

(c)	Explain why a NAND gate bistable could be described as having active low inputs.

..................................................................................................................................

..................................................................................................................................

(d)	What happens if both the Set input and Reset inputs are both connected to 0V at the 		same time?

..................................................................................................................................

..................................................................................................................................

(e)	Explain why the circuit is called 'Bistable'.

..................................................................................................................................

..................................................................................................................................

(f)	State and explain two uses of a bistable latch.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(2).	The D-Type Flip-Flop.

The aim of this assignment is to understand the D-type flip-flop and to become familiar with its applications.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a D-type Flop-Flop,

Explain how a D-type Flop-Flop works.

Know some applications of a D-type Flop-Flop.

You will need to use the Support Booklet.  The D-type flip-flops are contained in a 4013 IC.

1).	(a)	Draw a diagram to show the pin-out of the 4013 IC.

(b)	State the meaning of the following symbols used with D-type flip-flops:

D	..................................................................................................................................

S	..................................................................................................................................

CK	..................................................................................................................................

R	..................................................................................................................................

Q	..................................................................................................................................

Q	..................................................................................................................................

(c)	Use the support booklet to explain how the D-type flip-flop functions.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

2).	(a)	Use the protoboard diagram below to show how you can set up a D-type flip-flop 			from a 4013 IC.  Connect the D, CK, S and R inputs of one of the D-type flip-flops 		to 0V via 100k resistors.  Connect LEDs from the outputs to 0V via 1k resistors.

Remember to connect all of the inputs of the unused D-type flip-flop to 0V.

(b)	Explain what happens when:-

S is taken to logic 1

..................................................................................................................................

..................................................................................................................................

R is taken to logic 1

..................................................................................................................................

..................................................................................................................................

D is taken to logic 1

..................................................................................................................................

..................................................................................................................................

CK is taken to logic 1

..................................................................................................................................

..................................................................................................................................

D is taken to logic 1 and CK is momentarily connected to logic 1

..................................................................................................................................

..................................................................................................................................

(c)	Explain why this D-type flip-flop is said to be rising edge triggered.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

3).	(a)	In the space below use the support booklet to draw a diagram to show how D-type 		flip-flops can be used to form a 4-bit data latch.

(b)	Explain how the 4-bit data latch operates.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Describe a use for a data latch

..................................................................................................................................

..................................................................................................................................

(3).	The Shift Register.

The aim of this assignment is to understand the shift register and to become familiar with its applications.

By the end of this task you will be able to:-

Recognise and draw the circuit diagram of a Shift Register,

Explain how a Shift Register works.

Know some applications of a Shift register.

You will need to use the Support Booklet.  A 4 bit shift register can be constructed from the D-type flip-flops that are contained in two 4013 ICs.

1).	(a)	Use the Support Booklet to draw the circuit diagram of a 4-bit shift register using 			D-type flip-flops.

(b)	Explain how the shift register functions.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(c)	Explain how a shift register can convert parallel data into serial data.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(d)	Explain how a shift register can be used to convert serial data into parallel data.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

(e)	State and explain two uses for shift registers.

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

..................................................................................................................................

2).	(a)	If there is time, try to construct a 4-bit shift register.  Use the PROTOBOARD 			planning sheet below to help you set out the components.