NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

MOBILE

COMMUNICATIONS

PACKAGE

(1).	The General Structure of the Mobile Telephone System.

(2).	The Cellular Telephone System.

(3).	Calculation of the maximum number of users in a cell.

(4).	Repeaters, Regenerators, Cellular and Frequency reuse.

(5)	Modern uses of Mobile Communications.

F).(1).	The General Structure of the Mobile Telephone System.

The aim of this assignment is to develop an understanding of the general structure of the mobile telephone system.  Use the Support Booklet to help you complete the following assignment.

1).	(a)	Explain what is meant by 'base-station'.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(b)	How do the base-stations communicate with the mobile telephone?

.........................................................................................................................................

(c)	How do the base-stations communicate with other base stations?

.........................................................................................................................................

(d)	How do the base stations communicate with land telephones?

.........................................................................................................................................

(e)	What are the essential sub systems in a mobile telephone?

.........................................................................................................................................

.........................................................................................................................................

(f)	Describe the information path between a mobile telephone and a land telephone.

.........................................................................................................................................

.........................................................................................................................................

(g)	Describe two possible information paths between a mobile telephone and another 			mobile telephone some way from the first. (100km +)

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

F).(2).	The Cellular Telephone System.

The aim of this assignment is to develop an understanding of the Cellular Telephone System.

Use the Support Booklet to help you complete the following assignment.

1).	(a)	Explain what is meant by a 'cell'.

.........................................................................................................................................

.........................................................................................................................................

(b)	(i)	State the typical range of sizes of a 'cell'.

.........................................................................................................................................

(ii)	What factors determine the size of a cell?

.........................................................................................................................................

.........................................................................................................................................

(c)	What are the  radio frequencies bands used for mobile telephones.

.........................................................................................................................................

(d)	What property of the radio waves of these frequency bands enables a large number of 		mobile telephones to be used within a restricted frequency allocation?

.........................................................................................................................................

.........................................................................................................................................

(e)	What is the frequency allocation for GSM, DCS1800 and 3G telephones.

.........................................................................................................................................

.........................................................................................................................................

(f)	With GSM, how is frequency multiplexing used to maximise the number of

available mobile telephone channels within the frequency allocation?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(g)	With GSM, how is time division multiplexing used to maximise the number of 			available telephone channels within the frequency allocation?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

2).	(a)	Explain why there is a restriction in the range of frequencies that can be used in 			adjacent cells.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(b)	Draw a diagram to show how the frequency channels are allocated within a block of 		16 adjacent cells.

(c)	What commercially limits the number of mobile telephones that a telephone network 		operator can connect within a given cell?

.........................................................................................................................................

.........................................................................................................................................

F).(3).	Calculation of the maximum number of users in a cell.

The aim of this assignment is to develop an understanding of how to calculation the maximum number of users in a cell for the GSM telephone system.

Use the Support Booklet to help you complete the following assignment.

1).	(a)	State the bandwidth allocated for the Uplink for a GSM telephone.

.........................................................................................................................................

(b)	If this allocation is divided into 200kHz frequency channels, explain why there are 		124 possible frequency channels available.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(c)	How many data channels are fitted into each frequency channels?

.........................................................................................................................................

(d)	A mobile telephone operator is allocated 40 non-adjacent frequency channels for the 		entire network.

(i)	State, with a reason, the maximum number of frequency channels that he can 			allocate to each cell in his network?

.........................................................................................................................................

.........................................................................................................................................

(ii)	State, with a reason, the maximum number of subscribers that can be 				connected simultaneously in a cell.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(e)	The network operator finds that there are insufficient channels available for the 			number of subscribers wishing to use the network.  What are the options that he has 		for increasing the capacity?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

F).(4).	Repeaters, Regenerators, Cellular and Frequency re-use.

The aim of this assignment is to develop an understanding of the terms Repeaters, Regenerators, Cellular and Frequency re-use in connection with mobile communication systems.

Use the Support Booklet to help you complete the following assignment.

1).	(a)	(i)	Explain the function of a Repeater.

.........................................................................................................................................

.........................................................................................................................................

(ii)	What effect would a Repeater have on the waveform below.

Illustrate your answer with a suitable diagram.

.........................................................................................................................................

.........................................................................................................................................

(iii)	Explain why Repeaters have a limited use within a communication system.

.........................................................................................................................................

.........................................................................................................................................

(b)	(i)	Explain the function of a Regenerator.

.........................................................................................................................................

.........................................................................................................................................

(ii)	What effect would a Regenerator have on the waveform below.

Illustrate your answer with a suitable diagram.

.........................................................................................................................................

.........................................................................................................................................

(iii)	Explain why Regenerators are of much more use than Repeaters within a 				communication system.

.........................................................................................................................................

.........................................................................................................................................

(c)	(i)	Why are mobile telephone systems said to be cellular?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(ii)	Draw a diagram to illustrate the cellular nature of mobile telephone networks.

(d)	(i)	Explain what is meant by frequency re-use.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(ii)	Explain why frequency re-use is essential with the present mobile telephone 			system.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(e)	State and explain three ways in which the current GMS and DCS1800 				telephone systems maximise frequency re-use.

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

F).(5).	Modern uses of Mobile Communications.

The aim of this assignment is to develop an understanding of the Modern uses of Mobile Communications.  Use the Support Booklet to help you complete the following assignment.

1).	State and briefly describe the uses and problems caused by mobile telephones in each of 	the following areas of modern life.

(a)	Business;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(b)	Work;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(c)	Crime;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(d)	Emergencies;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(e)	Advertising;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(f)	Home;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(g)	News;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(h)	Information;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

(i)	Leisure;

Uses......................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

Problems...............................................................................................................

.........................................................................................................................................

.........................................................................................................................................

2).	a).	Summarise what other services are currently available using mobile telephones other 		than speech

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

b).	What other services are likely to become available with the 3G mobile telephones?

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................

.........................................................................................................................................