NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

P2T2

CONTROLLING

AND USING

ELECTRIC CURRENT

(1).	Electric current, potential difference and resistance.

(2 & 3).	Electric circuits and devices.

(4)	Electrical energy and power.

1).	Electric current, potential difference and resistance.

The aim of this assignment is to gain an understanding of the terms Electric current, potential difference and resistance.

Use the Internet and Revision Guide to help you complete the following tasks

A).	(a)	(i)	Draw the symbol for an ammeter

(ii)	Draw a circuit diagram to show how you would measure the current passing 			through a lamp.

(iii)	Explain whether it matters if the ammeter is before or after the lamp.

........................................................................................................................................

........................................................................................................................................

(b)	(i)	What happens to the electric current in the diagram below?

........................................................................................................................................

........................................................................................................................................

(ii)	Summarise what happens to the electric current entering a junction.

(Think about traffic at an island)

........................................................................................................................................

........................................................................................................................................

(c)	(i)	What is an electric current?

........................................................................................................................................

........................................................................................................................................

(ii)	What causes an electric current to flow around a circuit?

........................................................................................................................................

(iii)	In the circuit below, what will happen if the potential difference of the power 			supply is increased?

........................................................................................................................................

(iv)	Summarise the relationship between the potential difference of a source and 			the current passing through a circuit

........................................................................................................................................

........................................................................................................................................

B).	(a)	(i)	Draw the symbol for a voltmeter.

(ii)	Draw a circuit diagram to show how you would measure the potential 				difference across a lamp.

(iii)	State the units of potential difference.

........................................................................................................................................

(b)	(i)	State the relationship between potential difference, charge and energy.

........................................................................................................................................

........................................................................................................................................

........................................................................................................................................

(ii)	State alternative units for potential difference.

........................................................................................................................................

(c)	What limits the flow of electric current through a circuit?

........................................................................................................................................

........................................................................................................................................

2 & 3).	Electric circuits and devices.

The aim of this assignment is to gain an understanding of how devices behave in an electric circuit.

Use the Internet and Revision Guide to help you complete the following tasks

A).	You will need the following equipment:-

1x	variable power supply,

1x	10A ammeter,

1x	20V voltmeter,

1x	a mounted wire wound resistor,

5x	connecting leads.

Use the equipment to make up the circuit shown in the diagram opposite.

BEFORE switching on, check with your supervisor that it is connected correctly.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
| 0 |  |  |
| 1.0 |  |  |
| 2.0 |  |  |
| 3.0 |  |  |
| 4.0 |  |  |
| 5.0 |  |  |
| 6.0 |  |  |
| 7.0 |  |  |
| 8.0 |  |  |
| 9.0 |  |  |
| 10.0 |  |  |

(a)	Adjust the rheostat until the voltmeter reads each of the values in turn shown in the 		table and note down the reading on the ammeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

(b)	Plot a graph of your results on the graph paper, with p.d. on the vertical (y) axis and 		current on the horizontal (x) axis.

Draw the best straight line that will go through the points that you have plotted.

(c)	Find the gradient of your graph and record your answer on the graph paper.

(d)	What do you notice about the gradient of your graph and the values in the last 			column of the table?

................................................................................................................................................

CONCLUSION.

The graph is a straight line passing through the origin and shows that the current through the resistor is proportional to the voltage across it, (so long as the temperature remains constant.)

Ohm's Law calculations.

Ohm's law can be placed into a Magic Triangle

to help with its re-arrangement.

The magic triangle is shown opposite.

To find a formula, cover up the letter that you require and the formula can be read from the triangle.

e.g., to find I, cover the letter I and then I = V/R.

Questions.

These are to provide practice at using Ohm's Law.

(a)	Calculate the resistance of each resistor.

(1).	V = 6V	I = 2A		R =

(2).	V = 12V	I = 3A		R =

(3).	V = 2V	I = 4A		R =

(4).	V = 7V	I = 3.5A	R =

(5).	V = 8V	I = 1A		R =

(6).	V = 1V	I = 10A	R =

(7).	V = 9.2V	I = 0.4A	R =

(8).	V = 1000V	I = 2.5A	R =

(9).	V = 25V	I = 0.05A	R =

(10).	V = 0.01V	I = 10A	R =

(b)	Calculate the p.d. V across each resistor.

(11).	I = 1A		R = 7W	V =

(12).	I = 3A		R = 4W	V =

(13).	I = 2A		R = 19W	V =

(14).	I = 2A		R = 4W	V =

(15).	I = 15A	R = 5W	V =

(16).	I = 2.5A	R = 10W	V =

(17).	I = 1.1A	R = 3W	V =

(18).	I = 4.7A	R = 2.1W	V =

(19).	I = 13.2A	R = 4.7W	V =

(20).	I = 0.005A	R = 3.4W	V =

(c)	Calculate the current through each resistor.

(21).	V = 4V	R = 2W	I =

(22).	V = 20V	R = 2W	I =

(23).	V = 15V	R = 7.5W	I =

(24).	V = 10V	R = 20W	I =

(25).	V = 4V	R = 4W	I =

(26).	V = 2.2V	R = 1.1W	I =

(27).	V = 9.6V	R = 3.2W	I =

(28).	V = 0.07V	R = 0.28W	I =

(29).	V = 0.1V	R = 0.001W	I =

(30).	V = 0.06V	R = 0.6W	I =

HOMEWORK

Complete the Online assignment  OHM’S LAW.  Choose the Level 2 (GCSE) option.

Print out the results sheet and hand it in when completed.

B).	Most devices do not have a linear relationship between potential difference and current like 	resistors.  These experiments are about investigating the relationship for two common 	devices, a lamp and a diode.

You will need the following equipment:-

1x	unregulated 6V power supply,

1x	10A ammeter,

1x	20V voltmeter,

1x	12V lamp (24/36W),

5x	connecting leads.

Use the equipment to make up the circuit shown in the diagram opposite.

BEFORE switching on, check with your supervisor that it is connected correctly.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
| 0 |  |  |
| 0.5 |  |  |
| 1.0 |  |  |
| 1.5 |  |  |
| 2.0 |  |  |
| 2.5 |  |  |
| 3.0 |  |  |
| 3.5 |  |  |
| 4.0 |  |  |
| 4.5 |  |  |
| 5.0 |  |  |

(a)	Adjust the rheostat until the voltmeter reads each of the values in turn shown in the 		table and note down the reading on the ammeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

(b)	Plot a graph of your results on the graph paper, with current. on the vertical (y) axis 		and potential difference on the horizontal (x) axis.

Draw the best curved line that will go through the points that you have plotted.

(c)	What change has occurred to the filament wire in the lamp that did not occur to the 		wire in the resistor?

................................................................................................................................................

................................................................................................................................................

(d)	By looking at the last column in the table and the graph, what has happened to the 			resistance of the filament?

................................................................................................................................................

................................................................................................................................................

(e)	State the conditions needed for Ohm's Law to be valid for a wire carrying an electric 		current.

................................................................................................................................................

................................................................................................................................................

C).	(a)	Using the same circuit as for the lamp, replace the lamp with a diode.

NOTE – YOU ARE SETTING THE CURRENT THIS TIME.

Adjust the rheostat until the ammeter reads each of the values in turn shown in the 		table and note down the reading on the voltmeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

If there is no current flowing try reversing the connections to the diode.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
|  | 0 |  |
|  | 0.1 |  |
|  | 0.2 |  |
|  | 0.3 |  |
|  | 0.4 |  |
|  | 0.5 |  |
|  | 0.6 |  |
|  | 0.7 |  |
|  | 0.8 |  |
|  | 0.9 |  |
|  | 1.0 |  |

(b)	Plot a graph of your results on the graph paper, with current on the vertical (y) axis 		and potential difference on the horizontal (x) axis.

Draw the best curved line that will go through the points that you have plotted.

(c)	Summarise the results and describe the shape of the graph for the diode.

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

(d)	Describe a use for a diode.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

D).	(a)	A Light Dependent Resistor, LDR, is a device whose resistance varies with light 			intensity.

(i)	Draw the symbol for an LDR

(ii)	Connect a LDR to a digital meter set to read resistance and measure its 				maximum and minimum resistance.  Record these values.

................................................................................................................................................

................................................................................................................................................

(iii)	Summarise the operation of an LDR.

................................................................................................................................................

................................................................................................................................................

(b)	A thermistor is a device whose resistance varies with temperature.

(i)	Draw the symbol for a thermistor.

(ii)	Connect a thermistor to a digital meter set to read resistance and describe 				what happens when it is warmed and cooled.

................................................................................................................................................

................................................................................................................................................

(iii)	Summarise the operation of a thermistor.

................................................................................................................................................

................................................................................................................................................

4).	Electrical energy and power.

The aim of this assignment is to revise electrical energy and power and to calculate quantities.

Use the Internet and Revision Guide to help you complete the following tasks

A).	(a)	What happens when a current passes through a resistor.

(Resistor video)

........................................................................................................................................

........................................................................................................................................

........................................................................................................................................

(b)	Explain why this happens.  Use the diagram below to help.

........................................................................................................................................

........................................................................................................................................

........................................................................................................................................

........................................................................................................................................

(c)	State and explain two uses of the heating effect of an electric current.

(i)	..............................................................................................................

........................................................................................................................................

........................................................................................................................................

(ii)	..............................................................................................................

........................................................................................................................................

........................................................................................................................................

(d)	State and explain two problems of the heating effect of an electric current.

(i)	..............................................................................................................

........................................................................................................................................

........................................................................................................................................

(ii)	..............................................................................................................

........................................................................................................................................

........................................................................................................................................

B).	(a)	If 1 coulomb passes through a resistor in 1 second, what will be the current?

........................................................................................................................................

(b)	If 1 coulomb passes through a resistor in 1 second when there is a potential 			difference across the resistor of one volt, what will be the energy given to the 			resistor.

........................................................................................................................................

........................................................................................................................................

(c)	How much energy will be given to the resistor in the diagram below in one second.

........................................................................................................................................

........................................................................................................................................

(d)	How much energy will be given to the resistor in the diagram below in t seconds.

........................................................................................................................................

(e)	State a formula for electric power.  Include the units.

........................................................................................................................................

HOMEWORK

Complete the Online assignment  Electric Power.  Choose the Level 2 (GCSE) option.

Print out the results sheet and hand it in when completed.

NOTES