NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

BASIC

ELECTRICITY

PACKAGE

(11).	What is electricity?  and Ohm's Law

(12)	Series and parallel circuits.

(13)	Electrical power and energy.

(11).			What is electricity?

The aim of this assignment is to develop an understanding of what electricity is.

Use your Text Book to help you complete the following tasks.

A).	Read through the passage below and answer the following questions.

Electricity is the result of the movement of very small quantities of electric charge called ELECTRONS.  When electrons move, an electric current is produced which is measured in AMPs by an AMMETER.  Electric current is defined as the rate of change of charge.  I = ΔQ / Δt.  (The number of electrons per second).

To measure a current the ammeter has to be connected so that the current has to flow THROUGH the ammeter.  It has to be connected in SERIES.

The energy that electrons give out when going through a circuit is measured by their VOLTAGE (potential difference, p.d.) in VOLTs.  A VOLTMETER is the instrument used to measure voltage and is connected ACROSS the circuit.  It has to be connected in PARALLEL.  Potential difference is defined as the change in energy per unit charge.  V = ΔW / ΔQ.  (The energy that each electron has).

An electric current can only flow around a circuit when the electrons have energy, i.e., when there is a potential difference.

Electrons are never LOST in a circuit.  All of the electrons that start to flow around a circuit from a battery end up entering the other side of the battery.  Electrons do loose energy by the work that they do in going through the various parts of the circuit.

QUESTIONS.

a).	(i)	Explain what is meant by an electric current.

.............................................................................................................................................................

.............................................................................................................................................................

(ii)	State the units of electric current.

.............................................................................................................................................................

b).	What causes a current to flow around a circuit?

.............................................................................................................................................................

c).	(i)	Explain how you would measure an electric current.

.............................................................................................................................................................

.............................................................................................................................................................

(ii)	Draw a circuit diagram to show how you would connect an ammeter to measure the 		current flowing through a lamp from a battery.

d).	(i)	Explain what is meant by a potential difference.

.............................................................................................................................................................

.............................................................................................................................................................

(ii)	What units is potential difference measured in?

.............................................................................................................................................................

e).	(i)	Explain how you would measure a potential difference.

.............................................................................................................................................................

.............................................................................................................................................................

(ii)	Draw a circuit diagram to show how you would connect a voltmeter to measure the 		potential difference across a lamp.

Ohm’s Law.

The aim of this task is to investigate how the current flowing through a piece of wire varies with the voltage across it.  The law that results from this investigation is a very important, and underpins ALL electrical and electronics calculations.

Use your Text Book to help you complete the following tasks.

B).	You will need the following equipment:-

1x	variable power supply,

1x	10A ammeter,

1x	20V voltmeter,

1x	a mounted wire wound resistor,

5x	connecting leads.

Use the equipment to make up the circuit shown in the diagram opposite.

BEFORE switching on, check with your supervisor that it is connected correctly.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
| 0 |  |  |
| 1.0 |  |  |
| 2.0 |  |  |
| 3.0 |  |  |
| 4.0 |  |  |
| 5.0 |  |  |
| 6.0 |  |  |
| 7.0 |  |  |
| 8.0 |  |  |
| 9.0 |  |  |
| 10.0 |  |  |

(a)	Adjust the rheostat until the voltmeter reads each of the values in turn shown in the 		table and note down the reading on the ammeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

(b)	Plot a graph of your results on the graph paper, with p.d. on the vertical (y) axis and 		current on the horizontal (x) axis.

Draw the best straight line that will go through the points that you have plotted.

(c)	Find the gradient of your graph and record your answer on the graph paper.

(d)	What do you notice about the gradient of your graph and the values in the last 			column of the table?

................................................................................................................................................

................................................................................................................................................

CONCLUSION.

The graph is a straight line passing through the origin and shows that the current through the resistor is proportional to the voltage across it, (so long as the temperature remains constant.)

Ohm's Law calculations.

Ohm's law can be placed into a Magic Triangle

to help with its re-arrangement.

The magic triangle is shown opposite.

To find a formula, cover up the letter that you require and the formula can be read from the triangle.

e.g., to find I, cover the letter I and then I = V/R.

Questions.

These are to provide practice at using Ohm's Law.

A	Calculate the resistance of each resistor.

(1).	V = 6V	I = 2A		R =

(2).	V = 12V	I = 3A		R =

(3).	V = 2V	I = 4A		R =

(4).	V = 7V	I = 3.5A	R =

(5).	V = 8V	I = 1A		R =

(6).	V = 1V	I = 10A	R =

(7).	V = 9.2V	I = 0.4A	R =

(8).	V = 1000V	I = 2.5A	R =

(9).	V = 25V	I = 0.05A	R =

(10).	V = 0.01V	I = 10A	R =

B.	Calculate the p.d. V across each resistor.

(11).	I = 1A		R = 7W	V =

(12).	I = 3A		R = 4W	V =

(13).	I = 2A		R = 19W	V =

(14).	I = 2A		R = 4W	V =

(15).	I = 15A	R = 5W	V =

(16).	I = 2.5A	R = 10W	V =

(17).	I = 1.1A	R = 3W	V =

(18).	I = 4.7A	R = 2.1W	V =

(19).	I = 13.2A	R = 4.7W	V =

(20).	I = 0.005A	R = 3.4W	V =

C.	Calculate the current through each resistor.

(21).	V = 4V	R = 2W	I =

(22).	V = 20V	R = 2W	I =

(23).	V = 15V	R = 7.5W	I =

(24).	V = 10V	R = 20W	I =

(25).	V = 4V	R = 4W	I =

(26).	V = 2.2V	R = 1.1W	I =

(27).	V = 9.6V	R = 3.2W	I =

(28).	V = 0.07V	R = 0.28W	I =

(29).	V = 0.1V	R = 0.001W	I =

(30).	V = 0.06V	R = 0.6W	I =

More Ohm's Law calculations.

Complete the missing values in the table below.

| VOLTAGE (V) | CURRENT (A) | RESISTANCE () |
| --- | --- | --- |
|  | 2 | 5 |
| 2.5 | 0.5 |  |
| 240 |  | 960 |
|  | 13.2 | 4.7 |
| 25.0 | 0.05 |  |
| 9.2 |  | 0.4 |
| 3.3 | 0.16 |  |
|  | 0.000023 | 56000 |
| 12.0 |  | 680 |
|  | 1.3x10-5 | 82000 |

The circuit shown below is connected and the meters give the readings shown.

a).	What is the battery voltage?

................................................................................................................................................

b).	What is the total current flowing from the battery?

................................................................................................................................................

c).	What is the total resistance of the circuit?

................................................................................................................................................

d).	What is the resistance of R2?

................................................................................................................................................

................................................................................................................................................

e).	What current flows through the lamp?

................................................................................................................................................

f).	What is the voltage across the lamp?

................................................................................................................................................

g).	What is the resistance of the lamp?

................................................................................................................................................

................................................................................................................................................

h).	What is the resistance of R1?

................................................................................................................................................

................................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Ohm’s Law”.

There are two options for this exercise, Level 2 (GCSE) and Level 3 (AS).

Choose the most appropriate level and then print out and hand in your Score sheet.

(Level 2 exercises will be scored as a mark out of 20, Level 3 will be scaled out of 30)

(12).			Series and Parallel circuits.

The aim of this assignment is to learn the difference between series and parallel circuits and provide practice at calculating resistor combinations.

Use your text book to help you complete the following tasks.

A).	(a)	Draw the circuit diagram of two lamps connected in series and operating from a 			battery.

(b)	Describe the properties of a series circuit.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(c)	Draw the circuit diagram of two lamps connected in parallel and operating from a 			battery.

(b)	Describe the properties of a parallel circuit.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

Resistors in Series

B).	(a)	Draw a circuit diagram of two resistors in series.

Label the resistors R1 and R2 and show mathematically, how you would find the total		combined resistance.

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

(b)	Summarise the result.

................................................................................................................................................

................................................................................................................................................

C).	Find the combined resistance RT of the following resistors when connected in series:-

a).	5W	and	10W		RT =

b).	100W	and	220W		RT =

c).	500W	and	1kW		RT =

d).	4.7kW	and	2.7kW		RT =

e).	8.2kW	and	3.9kW		RT =

Connecting resistors in series increases the total resistance because, when a current passes through the combination, each resistor opposes the flow of electrons.

Homework Assignment.

Complete the Homework Exercise “Resistor Combinations”.

Choose Option 1, “Resistors in Series”

Print out and hand in your Score sheet.

Resistors in Parallel

D).	(a)	Draw a circuit diagram of two resistors in parallel.

Label the resistors R1 and R2 and show mathematically, how you would find the total 		combined resistance.

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

...............................................................................

(b)	Summarise the result.

................................................................................................................................................

................................................................................................................................................

E).	Find the total resistance of each of these parallel pairs:-

a).	4W	and	12W		RT =

b).	50W	and	200W		RT =

c).	220W	and	220W		RT =

d).	4.7kW	and	2.7kW		RT =

e).	56kW	and	68kW		RT =

Homework Assignment.

Complete the Homework Exercise “Resistor Combinations”.

Choose Option 2, “Resistors in Parallel”

Print out and hand in your Score sheet.

Combinations of resistors in Series and Parallel

F).	A 100W resistor is connected in parallel with a series combination of two 25W resistors.

(a)	Draw a diagram to show how the resistors are connected.

b).	Calculate the total resistance of the combination.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

c).	If a 12V battery is connected across the combination, what current will flow?

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

More combinations of resistors.

Find the total resistance in each case, i.e. find the value of the single resistor which would effectively replace each network of resistors.  Show your workings.

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

RT =

Homework Assignment.

Complete the Homework Exercise “Resistor Combinations”.

Choose Option 3, “Series and Parallel resistor combinations”.

Print out and hand in your Score sheet.

(13).			Electrical power and energy.

The aim of this assignment is to learn how to calculate electrical power and energy.

Use your text book to help you complete the following tasks.

A).	(a)	How much energy is used when an electron moves through
		one volt?

.............................................................................................................

(b)	Now much energy is used if N electrons move through a 			voltage V?

.............................................................................................................

(c)	Express the formula in (b) in terms of the charge moved.

................................................................................................................................................

(d)	If this change occurs in a time t, what is the formula for electrical power?

................................................................................................................................................

(e)	What is the unit of power?

................................................................................................................................................

B).	(a)	Put the above formula for electrical

power into a magic triangle.

(b)	If a car headlamp has a current flowing of 5A when connected to 12V, what power is 		being used?

................................................................................................................................................

................................................................................................................................................

C).	What is the power rating of a soldering iron which has a current of 2A when connected to a 	24V supply?

................................................................................................................................................

................................................................................................................................................

D)	(a).	What current passes through a 1000W electric fire operating from 230V mains?

................................................................................................................................................

................................................................................................................................................

(b).	What is the resistance of the electric fire element?

................................................................................................................................................

................................................................................................................................................

(c).	What value fuse would you use in the plug of the electric fire?

................................................................................................................................................

E)	(a).	What current passes through a 60W lamp operating from 230V mains?

................................................................................................................................................

(b).	What is the resistance of the filament of the lamp?

................................................................................................................................................

................................................................................................................................................

F).	A voltage of 22V is connected across a 220W resistor.

(a).	What current passes through the resistor?

................................................................................................................................................

................................................................................................................................................

(b).	What is the power used by the resistor?

................................................................................................................................................

................................................................................................................................................

G).	(a)	What is a fuse?

................................................................................................................................................

................................................................................................................................................

(b)	What is the purpose of a fuse

................................................................................................................................................

................................................................................................................................................

(c)	What is the modern equivalent of a fuse?

................................................................................................................................................

................................................................................................................................................

(d)	What is an RCD?

................................................................................................................................................

................................................................................................................................................

H).	Calculate the current passing when each of the following is connected to a 230V mains 	supply and state the required fuse for the mains plugs:-

(a).	a 2kW kettle,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

(b).	a 60W lamp,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

(c).	a 250W television,

Current ...................................................................................................................................

Fuse ........................................................................................................................................

(d).	a 700W microwave cooker.

Current ...................................................................................................................................

Fuse ........................................................................................................................................

I).	A LED is to be operated from a 12V supply, by connecting it in series with a resistor.

(a).	Draw a circuit diagram of a LED connected in series with a resistor.

(b).	If the LED needs a voltage of 2V across it, what is the voltage across the series 			resistor when connected to the 12V supply?

...............................................................................................................................................

...............................................................................................................................................

(c).	If a current of 25mA passes through the LED, what current passes through the 			resistor?

...............................................................................................................................................

...............................................................................................................................................

d).	Calculate the required value of the series resistor.

...............................................................................................................................................

...............................................................................................................................................

e).	Calculate the power dissipated in the series resistor.

...............................................................................................................................................

...............................................................................................................................................

f).	Calculate the power dissipated in the LED.

...............................................................................................................................................

...............................................................................................................................................

Homework Assignment.

Complete the Homework Exercise “Power Calculations”.

There are two options for this exercise, Level 2 (GCSE) and Level 3 (AS).

Choose the most appropriate level and then print out and hand in your Score sheet.

(Level 2 exercises will be scored as a mark out of 20, Level 3 will be scaled out of 30)

Complete the Online True/False “Ohm’s Law Test”.

Print out and hand in the Score sheet.

J).	(a)	Derive a formula for electrical energy from the formula for electric power.

...............................................................................................................................................

...............................................................................................................................................

(b)	Calculate how much energy is used by a 1kW electric fire operating for 1 hour.

...............................................................................................................................................

...............................................................................................................................................

(c)	What is the name for this quantity of energy?

...............................................................................................................................................

(d)	This quantity of energy is used by electrical suppliers to charge for electricity.

What is the cost of one such unit of electrical energy?

...............................................................................................................................................

(e)	Calculate the cost of running a 11W compact fluorescent lamp for 24 hours.

...............................................................................................................................................

...............................................................................................................................................

(f)	Calculate the cost of using a 9kW shower for 15 minutes.

...............................................................................................................................................

...............................................................................................................................................

(g)	Estimate the cost per day of heating a student flat with a 2kW electrical heater.

...............................................................................................................................................

...............................................................................................................................................

K).	(a)	Explain what is meant by the term Efficiency.

...............................................................................................................................................

...............................................................................................................................................

(b)	State a formula for efficiency.

...............................................................................................................................................

...............................................................................................................................................

(c)	An 700W electric motor produces 500W of mechanical power.

Calculate its efficiency.

...............................................................................................................................................

...............................................................................................................................................

L).	(a)	An electric car is powered by a 25kW electric motor.  If the Lithium polymer battery 		has a potential difference of 250V, calculate the current passing into the motor.

...............................................................................................................................................

...............................................................................................................................................

(b)	The terminal velocity of the car is 45ms-1 when the air resistance is 450N.

Calculate the efficiency of the electric motor.

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

...............................................................................................................................................

(c)	What will happen to the energy not converted into mechanical work?

...............................................................................................................................................

NOTES