NAME:......................................................................

DATE ISSUED:........................................................

COMPLETION DATE:............................................

FEEDBACK:

FURTHER

ELECTRICITY

PACKAGE

(14).	I-V curves for non-ohmic conductors

(15)	EMF and internal resistance.

(16)	Conductors, semiconductors and insulators.

(17)	Potential divider circuit.

(14).			I-V curves for non-ohmic conductors.

The aim of this task is to investigate how the current flowing through a non-ohmic conductor varies with the voltage across it.

Use your Text Book to help you complete the following tasks.

A).	You will need the following equipment:-

1x	variable power supply,

1x	10A ammeter,

1x	20V voltmeter,

1x	12V lamp (24/36W),

5x	connecting leads.

Use the equipment to make up the circuit shown in the diagram opposite.

BEFORE switching on, check with your supervisor that it is connected correctly.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
| 0 |  |  |
| 0.5 |  |  |
| 1.0 |  |  |
| 1.5 |  |  |
| 2.0 |  |  |
| 2.5 |  |  |
| 3.0 |  |  |
| 3.5 |  |  |
| 4.0 |  |  |
| 4.5 |  |  |
| 5.0 |  |  |

(a)	Adjust the rheostat until the voltmeter reads each of the values in turn shown in the 		table and note down the reading on the ammeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

(b)	Plot a graph of your results on the graph paper, with current. on the vertical (y) axis 		and potential difference on the horizontal (x) axis.

Draw the best curved line that will go through the points that you have plotted.

(c)	What change has occurred to the filament wire in the lamp that did not occur to the 		wire in the resistor?

................................................................................................................................................

................................................................................................................................................

(d)	By looking at the last column in the table and the graph, what has happened to the 			resistance of the filament?

................................................................................................................................................

................................................................................................................................................

(e)	State the conditions needed for Ohm's Law to be valid for a wire carrying an electric 		current.

................................................................................................................................................

................................................................................................................................................

B).	(a)	Using the same circuit as for the lamp, replace the lamp with a diode.

NOTE – YOU ARE SETTING THE CURRENT THIS TIME.

Adjust the rheostat until the ammeter reads each of the values in turn shown in the 		table and note down the reading on the voltmeter.

Divide the voltage by the current for each set of values in the table and record the 			answers in the last column of the table.

If there is no current flowing try reversing the connections to the diode.

| P.d. (V) | Current (A) | Voltage/Current |
| --- | --- | --- |
|  | 0 |  |
|  | 0.1 |  |
|  | 0.2 |  |
|  | 0.3 |  |
|  | 0.4 |  |
|  | 0.5 |  |
|  | 0.6 |  |
|  | 0.7 |  |
|  | 0.8 |  |
|  | 0.9 |  |
|  | 1.0 |  |

(b)	Plot a graph of your results on the graph paper, with current on the vertical (y) axis 		and potential difference on the horizontal (x) axis.

Draw the best curved line that will go through the points that you have plotted.

(c)	Summarise the results and describe the shape of the graph for the diode.

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

.................................................................................

(d)	Describe a use for a diode.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(15).			EMF and internal resistance.

The aim of this assignment is to know what is meant by the term EMF and to understand the concept of internal resistance.

Use your text book to help you complete the following tasks.

A).	(a)	If a direct short circuit is placed across a battery, according to Ohm’s law, what 			should be the current passing?

................................................................................................................................................

(b)	In practice, with a real battery, this does not happen.  Why not?

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(c)	Draw a circuit diagram to represent a real battery.

(d)	Explain what is meant by the term EMF.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(e)	Suggest a method of measuring the EMF of a battery.

................................................................................................................................................

................................................................................................................................................

B).	Measurement of the internal resistance of a D cell.

(a)	Connect a 2V digital voltmeter,

10A digital ammeter and a 16Ω 			rheostat

as shown in the circuit diagram 			opposite, but DO NOT connect the 			wire to the rheostat yet.

(b)	Measure the terminal p.d. of the cell and record your result.

Terminal p.d. = ............................................................................................

(c)	Set the rheostat to its largest resistance and then connect it into the circuit.

Adjust the rheostat to give you several (5+) different current readings and record 			these and the corresponding voltmeter reading in the table below.

You should start with a small current and then gradually decrease the rheostat to 			make more current flow

| P.d. (V) | Current (A) |
| --- | --- |

(d)	Plot a graph of your results on the graph paper, with potential difference on the 			vertical (y) axis and current on the horizontal (x) axis.

Draw the best straight line that will go through the points that you have plotted.

(e)	Calculate the gradient of your graph.

................................................................................................................................................

(f)	What quantity is represented by the intercept on the vertical axis?

................................................................................................................................................

(g)	What quantity is represented by the gradient?

................................................................................................................................................

(h)	State a formula that relates the emf of the cell with its internal resistance, current 			flowing and terminal potential difference.

................................................................................................................................................

C).	(a)	A battery has an emf of 9V and a short circuit current of 1.5A.

Calculate the internal resistance of the battery.

................................................................................................................................................

................................................................................................................................................

(b)	A car battery has an emf of 13.2V and a maximum starting current of 450A.

Calculate the internal resistance of the battery.

................................................................................................................................................

(c)	When a battery is supplying a current of 1.2A, its terminal p.d. is 4.8V.

When the battery is supplying a current of 2.0A, its terminal p.d. is 3.6V.
		Calculate:-

(i)	the emf of the battery.

................................................................................................................................................

(ii)	the internal resistance of the battery.

................................................................................................................................................

................................................................................................................................................

(d)	A lithium polymer battery, with an emf of 6.1V, has a short circuit current of 12A.

(i)	Calculate the internal resistance of the battery.

................................................................................................................................................

(ii)	Calculate the power dissipated in the internal resistance of the battery.

................................................................................................................................................

................................................................................................................................................

(iii)	Why is this dangerous?    (Video  Battery:-  Laptop fire)

................................................................................................................................................

................................................................................................................................................

(16).			Conductors, semiconductors and insulators.

The aim of this assignment is to learn how to be aware of the difference between conductors, semiconductors and insulators and to develop a model to explain these differences.

Use your text book to help you complete the following tasks.

A).	(a)	List four elements that conduct electricity.

................................................................................................................................................

(b)	What is the generic name for these elements?

................................................................................................................................................

(c)	Name an element that conducts electricity that does not belong to this generic group

................................................................................................................................................

B).	(a)	Describe, using a diagram, the atomic

structure of the main group of

elements that conduct electricity.

...........................................................................

...........................................................................

...........................................................................

(b)	Explain why the outer electron of each atom is only held weakly in place

................................................................................................................................................

................................................................................................................................................

(c)	By making additions to the diagram below, explain what is likely to happen to the 			outer electrons in block of the element.

................................................................................................................................................

................................................................................................................................................

(d)	What will happen to the outer electrons if the left hand side of the block is made 			negative and the right 	hand side is positive.

................................................................................................................................................

................................................................................................................................................

(e)	Explain the concept of resistance in terms of this model for a conductor.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(f)	At an atomic level, what two effects will occur if the block is heated.

................................................................................................................................................

................................................................................................................................................

(g)	Explain your results for the I-V curve for a lamp in terms of this model for a 			conductor.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(h)	At an atomic level, what two effects will occur if the block is cooled.

................................................................................................................................................

................................................................................................................................................

(i)	Explain the concept of superconductivity in terms of this model.

(Video  Physics – Superconductivity)

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

C).	(a)	Based on the model for a conductor, suggest a likely model for an insulator.

................................................................................................................................................

................................................................................................................................................

(b)	The structure of silicon is shown below.

Explain whether this material likely to be an insulator or a conductor at room 			temperature?

................................................................................................................................................

................................................................................................................................................

(c)	Suggest what might happen if the silicon is heated?

(Glass rod demonstration)

................................................................................................................................................

................................................................................................................................................

(d)	How can this be explained in terms of this model?

................................................................................................................................................

................................................................................................................................................

D).	(a)	What is likely to happen if there are impurities within the insulator?

................................................................................................................................................

................................................................................................................................................

(b)	Phosphorus contains five electrons in its outer shell.

Add a phosphorus atom to the diagram below and then explain the effect this will 			have on the silicon.

................................................................................................................................................

................................................................................................................................................

(c)	Aluminium contains three electrons in its outer shell.

Add an aluminium atom to the diagram below and then explain the effect this will 		have on the silicon.

................................................................................................................................................

................................................................................................................................................

E).	(a)	The number of ‘free’ electrons per unit volume in a material is called the number 			density and is given the symbol n.

Show that  I = n.e.v.A  where e is the charge on an electron, v is the drift velocity of 		the electrons (charge carriers) and A is the cross sectional area of the conductor.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(b)	The electron number density for copper is approximately 1028.  Calculate the drift 			velocity of the electrons if a current of 10A is flowing through a wire of cross 			sectional area 1mm2.

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

(c)	Suggest a experiment to verify this value.

(Electrophoresis animation)

.........................................................................

.........................................................................

.........................................................................

.........................................................................

.........................................................................

.........................................................................

F).	(a)	What factors are likely to affect the resistance of a material?

(Think about supermarket shopping!)

................................................................................................................................................

(b)	How might each of these factors affect the resistance?

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

Investigation of the factors affecting resistance.  You will need a Multimeter, a ruler and a soft pencil (2B+), and will involve using the carbon from the pencils to form resistors on paper.

Use 4mm plug leads and a multimeter set to measure resistance to make your measurements.

Ensure that you do not touch the metal parts of the plugs when making the measurements.

Ensure that you read and understand the instructions for this work BEFORE carrying out the experiments.

G).	(a)	Colour in circle A and circle F in the diagram below.

Set the multimeter to the 2M resistance range.

Hold one of the leads from the multimeter on circle A and the other on circle F.

Note that the resistance is too large to be measured.

(b)	Colour in the circles in between A and F.

Now draw a thick pencil line between circles A and B.

Ensure that the pencil line is dense.

Hold one of the leads from the multimeter on circle A and the other lead on circle B.

Record the resistance in the table.

Repeat the measurements for the other circles and record your measurements in the 		table.

|  | RESISTANCE M |
| --- | --- |
| A to B |  |
| A to C |  |
| A to D |  |
| A to E |  |
| A to F |  |

(c)	What effect has the pencil line had on the resistance between circles A and F?

................................................................................................................................................

(d)	What do you notice about the resistance of the pencil line?

................................................................................................................................................

................................................................................................................................................

H).	(a)	Colour in rectangles A and B in the diagram below.

Set the multimeter to the 2M resistance range.

Hold one of the leads from the multimeter on rectangle A and the other on 
		rectangle B.

Note that the resistance is too large to be measured.

(b)	Colour in the top bar in between A and B.

Hold one of the leads from the multimeter on rectangle A and the other lead on 			rectangle B.

Record the resistance in the table.

(c)	Now colour in the next bar down in between A and B.

Hold one of the leads from the multimeter on rectangle A and the other lead on 			rectangle B.

Record the resistance in the table.

(d)	Repeat this for each of the last three bars in between A and B

Record the resistance in the table.

| A to B | RESISTANCE M |
| --- | --- |
| Top bar |  |
| Top two bars |  |
| Top three bars |  |
| Top four bars |  |
| All of the bars |  |

(c)	What do you notice about how the thickness of the pencil line alters the resistance?

................................................................................................................................................

................................................................................................................................................

I).	(a)	Give a formula to summarise the results from these experiments as proportionalities.

................................................................................................................................................

(b)	The constant of proportionality is called ‘resistivity’.  State the units of resistivity.

................................................................................................................................................

(c)	A copper wire of length 10m and cross sectional area 1mm2 has a resistance
		of 0.17Ω.  Calculate the resistivity of copper.

................................................................................................................................................

................................................................................................................................................

(d)	Aluminium has a resistivity of 2.65x10-8Ωm.

Calculate the resistance of a 50m aluminium wire of radius 1.5mm.

................................................................................................................................................

................................................................................................................................................

(e)	When a 6V, 0.06A lamp is lit to full brightness, the tungsten filament reaches a 			temperature of 1400°C.  If the filament is 2cm long, calculate the thickness 			(diameter) of the filament.  (Resistivity of tungsten at 1400°C is 4.355x 10-7Ωm.

................................................................................................................................................

................................................................................................................................................

(f)	Electricity is supplied to a house through copper wires of diameter 6mm.  The live 		and neutral wires are each 50m long and the electricity supply is 230V.

When a 9kW shower is turned on, it is noticed that the lights in the house dim 			slightly.

(i)	Calculate the total resistance of the supply wires.

................................................................................................................................................

................................................................................................................................................

(ii)	Calculate the voltage lost across the supply wires.

................................................................................................................................................

................................................................................................................................................

(iii)	Explain why the lights dim when the shower is used.

................................................................................................................................................

(17).			Potential divider circuit.

The aim of this assignment is to learn how to calculate the output voltage and resistance for a potential divider.

Use your text book to help you complete the following tasks.

A).	Consider the circuit diagram below.

(a)	What is the total resistance across the supply voltage, Vin?

................................................................................................................................................

(b)	What is the current, I, passing through the resistors?

................................................................................................................................................

................................................................................................................................................

(c)	What is the voltage across R2, Vout?

................................................................................................................................................

................................................................................................................................................

(d)	Combine (b) and (c) together to form an equation which relates Vin and Vout.

................................................................................................................................................

................................................................................................................................................

(e)	If Vin is 12V, R1 10Ω, R2 is 15Ω, calculate the output voltage Vout.

(Draw a labelled circuit diagram)

..........................................................................

..........................................................................

..........................................................................

..........................................................................

B).	Complete the table below.

| Vin | Vout | R1 | R2 |
| --- | --- | --- | --- |
| 30V |  | 6.2Ω | 9.1Ω |
| 40V |  | 47Ω | 75Ω |
| 14V |  | 2Ω | 3Ω |
| 2V |  | 13Ω | 24Ω |
| 0.49V | 0.07V |  | 3300Ω |
| 560V | 250V |  | 1.3MΩ |
| 7.6kV | 4.0kV |  | 9.1MΩ |
| 670V | 191V | 300kΩ |  |
|  | 545V | 11kΩ | 240kΩ |
|  | 350V | 3.6MΩ | 4.7MΩ |

C).	(a)	A Light Dependent Resistor, LDR, is a device whose resistance varies with light 			intensity.

(i)	What is the symbol for an LDR?

(ii)	Connect a LDR to an digital meter set to read resistance and measure its 				maximum and minimum resistance.  Record these values.

................................................................................................................................................

................................................................................................................................................

(iii)	Summarise the operation of an LDR.

................................................................................................................................................

................................................................................................................................................

(b)	Draw a circuit diagram to show how a LDR could be included in a potential divider 		circuit so that as the light level increased, the output decreased.

(c)	Verify that your circuit is correct by setting up the circuit with a suitable value 			resistor, a 5V lab. power supply and a digital meter set to measure voltage.

What value resistor was needed to give an output voltage of approximately 2.5V in 		normal lab. lighting.

................................................................................................................................................

(d)	How could the circuit be re-arranged so that as the light level increased, the output 		voltage increased?  Draw the circuit diagram below, and verify your circuit is correct 		by constructing it.

D).	Repeat the experiments in section (3) but this time using a thermistor.

Investigate what happens to the resistance of a thermistor as its temperature changes.

Draw a circuit diagram show how a thermistor could be included in a potential divider 		circuit so that as the temperature rises, the output voltage rises.

Summarise your investigations below.

.........................................................................

.........................................................................

.........................................................................

.........................................................................

.........................................................................

.........................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

................................................................................................................................................

NOTES