µ §Sensor, Op-amp comparator, and output revision.

1).	For growing tropical plants it is necessary to ensure that the greenhouses are maintained at a 	minimum temperature at all times.  An electronic systems is used to achieve this.  The 		circuit diagram of the system used is shown below.

µ §

(a)	Mark onto the circuit diagram above the following sub-systems:

(i)	a voltage regulator sub-system,

(ii)	an input sub-system,

(iii)	a voltage divider sub system,

(iv)	an output sub-system

(v)	a comparator sub-system.

(5)

(b)	(i)	Explain why the voltage at point P is 12V.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	The 12V zener diode has a maximum power rating of 400mW.  Calculate the 			maximum current that can pass through this diode.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	Calculate the minimum value for resistor R.

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	Explain what value resistor you would choose from the E24 series.

..............................................................................................................................

..............................................................................................................................

(2)

(v)	Calculate a suitable power rating for this resistor.

..............................................................................................................................

..............................................................................................................................

(2)

(c)	Using the graph of the characteristic of the thermistor, state the resistance of the 			thermistor at the following temperatures:

(i)	10ºC  .........................................................................................................

(ii)	20ºC  .........................................................................................................

(iii)	25ºC  .........................................................................................................

(iv)	40ºC  .........................................................................................................

(v)	50ºC  .........................................................................................................

(5)

(d)	(i)	Calculate the voltage at point B.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	State, with a reason, the voltage at point A that will make the output of the 			op-amp change state.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	State, with a reason, the resistance of the thermistor that will make the output 			of the op-amp change state.

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	State, with a reason, the temperature of the thermistor that will make the 				output of the op-amp change state.

..............................................................................................................................

..............................................................................................................................

(2)

(v)	State and explain two reasons why an op-amp makes such an effective 				comparator.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(e)	(i)	State, with a reason, the voltage at Q when the thermistor is at a temperature 			of 22ºC.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	The LED emits blue light when the voltage at Q is 24V.  Estimate the voltage 			at the gate of the MOSFET when the LED is lit.

..............................................................................................................................

(1)

(iii)	The maximum current that can pass safely through the LED is 30mA.  				Calculate a suitable value for the resistor RR.

..............................................................................................................................

..............................................................................................................................

(2)

(f)	(i)	Label the relay in the circuit diagram with the word relay.

(1)

(ii)	State and explain three reasons why a relay is used in this system.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(iii)	Explain why the relay cannot be connected directly to the output of the
			op-amp

..............................................................................................................................

..............................................................................................................................

(1)

(iv)	State and explain three reasons why a MOSFET makes an ideal output 				buffer.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(v)	Explain the purpose of the diode labelled D.

..............................................................................................................................

..............................................................................................................................

(2)

(g)	(i)	Calculate the current that relay contacts must switch.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	Explain whether the relay contacts should be NO or NC types.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	State a suitable value for the fuse.

..............................................................................................................................

(1)

2).	In order to reduce the amount of vandalism occurring a local council decides to install an 	automatic lighting system which will turn on the street lights around a shopping precinct 	whenever the daylight falls below a certain level.  The circuit diagram of the electronic 	control system used is shown below.

µ §

(a)	Mark onto the circuit diagram above the following sub-systems:

(i)	a voltage regulator sub-system,

(ii)	an input sub-system,

(iii)	a voltage divider sub system,

(iv)	an output sub-system

(v)	a comparator sub-system.

(5)

(b)	(i)	What is the voltage at point P.

..............................................................................................................................

..............................................................................................................................

(1)

(ii)	The 9.1V zener diode has a maximum power rating of 400mW.  Calculate the 			maximum current that can pass through this diode.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	Calculate the minimum value for resistor R.

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	Explain what value resistor you would choose from the E24 series.

..............................................................................................................................

..............................................................................................................................

(2)

(v)	Calculate a suitable power rating for this resistor.

..............................................................................................................................

..............................................................................................................................

(2)

(c)	Using the graph of the characteristic of the LDR, state the resistance of the LDR at 		the following light levels:

(i)	10 lux  .......................................................................................................

(ii)	70 lux  .......................................................................................................

(iii)	200 lux  .....................................................................................................

(iv)	1000 lux  ...................................................................................................

(v)	4000 lux  ...................................................................................................

(5)

(d)	(i)	Calculate the voltage at point B.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	State, with a reason, the voltage at point A that will make the output of the 			op-amp change state.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	State, with a reason, the resistance of the LDR that will make the output of 			the op-amp change state.

..............................................................................................................................

..............................................................................................................................

(2)

(iv)	State, with a reason, the light level  that will make the output of the op-amp 			change state.

..............................................................................................................................

..............................................................................................................................

(2)

(v)	State and explain two reasons why an op-amp makes such an effective 				comparator.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(2)

(e)	(i)	State, with a reason, the voltage at Q when the light level falling onto the 				LDR is 2000 lux.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	What is the approximate base emitter voltage of a transistor that is 					conducting.

..............................................................................................................................

(1)

(iii)	The base current needed by the transistor is 2mA.  Calculate a suitable value 			for the resistor RR.

..............................................................................................................................

..............................................................................................................................

(2)

(f)	(i)	Label the relay in the circuit diagram with the word relay.

(1)

(ii)	State and explain three reasons why a relay is used in this system.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(iii)	Explain why the relay cannot be connected directly to the output of the
			op-amp

..............................................................................................................................

..............................................................................................................................

(1)

(iv)	State and explain three reasons why a MOSFET makes a better output 				buffer than a transistor.

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

..............................................................................................................................

(3)

(v)	Explain the purpose of the diode labelled D.

..............................................................................................................................

..............................................................................................................................

(2)

(g)	(i)	Calculate the current that relay contacts must switch.

..............................................................................................................................

..............................................................................................................................

(2)

(ii)	Explain whether the relay contacts should be NO or NC types.

..............................................................................................................................

..............................................................................................................................

(2)

(iii)	State a suitable value for the fuse.

..............................................................................................................................

(1)